Functional Design Specification

Initiative A- Standard Naming Convention

File Name:		Standard Naming Convention_V3.4_20160523

Version:	 		3.4

Status:			Re-vised

Date:			23 May 2016 (29 April 2013)

Author:			TCS / Vodacom

# Revision History

| Date | Version | Description | Author | Company |
| --- | --- | --- | --- | --- |
| 08 April 2013 | 1.0 | Initial Draft version for review | TCS | TCS |
| 29 April 2013 | 1.0 | Final version for review and sign-off | TCS | TCS |
| 27 May 2014 | 1.1 | Minor update | Vodacom | Vodacom |
| 14 July 2014 | 2.0 | Updates based on stakeholder feedback | Jacques Blaauw John Abington Marika du Plessis | Vodacom |
| 25 Aug 2014 | 2.1 | Updates based on stakeholder feedback | Marika du Plessis | Vodacom |
| 12 Sep 2014 | 2.2 | Updated approval list | Stefan Prinsloo | Vodacom |
| 10 Oct 2014 | 2.3 | Update diagrams | Marika du Plessis | Vodacom |
| 17 Oct 2014 | 2.4 | Update CDN Information | Marika du Plessis | Vodacom |
| 14 Nov 2014 | 2.5 | Include Labelling Products and Power Labels specification | Marika du Plessis | Vodacom |
| 25 Nov 2014 | 2.6 | Include address under Location attributes, Appendix update of CDN nodes | Marika du Plessis | Vodacom |
| 01-Dec-2014 | 2.7 | Updated based on review comments | Vodacom | Vodacom |
| 10-Apr-2015 | 2.8 | Updated new Coriant Node codes, ODF Port, Device Port and Patch Cord Naming to be aligned with ConnectMaster functionality | Vodacom | Vodacom |
| 20 Aug 2015 | 2.9 | Add Fibre Pole Naming and Pole Label Convention, Add rack types, correct one Coriant code | Marika du Plessis | Vodacom |
| 29 Sep 2015 | 3.0 | Add Object Types, Add Algorithm for Location Names with a multiple words, Add Circuit Functions code creation business rules. | Abhishek Kothari | Vodacom |
| 19 Oct 2015 | 3.1 | More detailed Circuit Function code calculation, Add Bandwidth to Value Mapping, Add Pole Manufacturer and Specifications, Add Lesotho Province (Districts) and Codes, confirmed ODF Patch Panel list | Marika du Plessis & Abhishek Kothari | Vodacom |
| 9 Feb 2016 | 3.2 | Update Pole Label specification | Marika du Plessis | Vodacom |
| 26 April 2016 | 3.3 | Update Grafoplast Cable Label Specification | Marika du Plessis | Vodacom |
| 23 May 2016 | 3.4 | Add Power Grid Areas Naming Convention | Marika du Plessis | Vodacom |

# Contributors

| Date | Version | Name | Designation | Company |
| --- | --- | --- | --- | --- |
| 29 April 2013 | 1.0 | TCS Team | Contractor | TCS |
| Jun-Aug 2014 | 1.2 | Adrian von Maltitz | NMG OSS | Vodacom |
| Jun-Aug 2014 | 1.2 | Antoni Gill | TX | Vodacom |
| Jun-Aug 2014 | 1.2 | Brad Phillips | RAN | Vodacom |
| Jun-Aug 2014 | 1.2 | Danie Knipe | RAN | Vodacom |
| Jun-Aug 2014 | 1.2 | David du Plessis | TX | Vodacom |
| Jun-Aug 2014 | 1.2 | Ian Dunleavy | NQ | Vodacom |
| Jun-Aug 2014 | 1.2 | Donna Kambarran | NQ | Vodacom |
| Jun-Aug 2014 | 1.2 | Fanie Van Der Merwe | PM | Vodacom |
| Jun-Aug 2014 | 1.2 | Gideon Schepers | NQ | Vodacom |
| Jun-Aug 2014 | 1.2 | Grant Bresler | RAN | Vodacom |
| Jun-Aug 2014 | 1.2 | Iedereez Mohamed | TX | Vodacom |
| Jun-Aug 2014 | 1.2 | Anton Meyer | TX | Vodacom |
| Jun-Aug 2014 | 1.2 | Ben Visser | RAN | Vodacom |
| Jun-Aug 2014 | 1.2 | Ignatius Ferreira | RAN | Vodacom |
| Jun-Aug 2014 | 1.2 | Johan Kleynhans | CORE | Vodacom |
| Jun-Aug 2014 | 1.2 | Leon Nadan | CORE | Vodacom |
| Jun-Aug 2014 | 1.2 | Lloyd Mphahlele | TX | Vodacom |
| Jun-Aug 2014 | 1.2 | Miles Yarrington | NMG OSS | Vodacom |
| Jun-Aug 2014 | 1.2 | Neliswa Sango | TX | Vodacom |
| Jun-Aug 2014 | 1.2 | Nesham Balmogim | FM | Vodacom |
| Jun-Aug 2014 | 1.2 | Neville Green | CORE | Vodacom |
| Jun-Aug 2014 | 1.2 | Regan Govender | VB / EBU | Vodacom |
| Jun-Aug 2014 | 1.2 | Chris Reid | CORE | Vodacom |
| Jun-Aug 2014 | 1.2 | Charl Louw | VB / EBU | Vodacom |
| Jun-Aug 2014 | 1.2 | Marius Beukes | VB / EBU | Vodacom |
| Jun-Aug 2014 | 1.2 | Surajh Seelajith | TX | Vodacom |
| Jun-Aug 2014 | 1.2 | Thome Theron | NMG OSS | Vodacom |
| Jun-Aug 2014 | 1.2 | Trevor Eckleton | TX | Vodacom |
| Jun-Aug 2014 | 1.2 | Ursula George | RAN | Vodacom |
| Jun-Oct 2014 | 1.3 | Fred Albrecht | CDN | Vodacom |
| Jun-Oct 2014 | 1.3 | Peet Uys | CDN | Vodacom |
| Jun-Oct 2014 | 1.3 | Jacques Hendricks | CDN | Vodacom |
| Jun-Oct 2014 | 1.3 | Jurgen du Plessis | CDN | Vodacom |

# Signoff

This table serves as an acceptance table and once signed by all parties, this document is deemed to be accepted.  The Detailed Design will be drawn up from this document and no additional changes shall be made that do not appear in this signed off document.

| Date | Vers | Name | Designation & Business Area / Internal Customer | Signature |
| --- | --- | --- | --- | --- |
|  | 1.2 | Andries Delport | Chief Technology Officer |  |
|  | 1.2 | Christoph Labuschagne | Network Operations Officer |  |
|  | 1.2 | Johann Pretorius | ME: Enterprise Technology |  |
|  | 1.2 | Shane Hibbard | ME: Gauteng Region |  |
|  | 1.2 | Andries Steenkamp | ME: Core & Transport Network Engineering |  |
|  | 1.2 | Alberts Breed | ME: KwaZulu Natal Region |  |
|  | 1.2 | Steven Barnwell | ME: Western Cape Region |  |
|  | 1.2 | Louisa van Beek | ME: Mpumalanga Region |  |
|  | 1.2 | Carol Hall | ME: Eastern Cape Region |  |
|  | 1.2 | Jorge Mendes | ME: Central Region |  |
|  | 1.2 | Thys Delport | ME: Network Programme Management |  |
|  | 1.2 | Henry Arnold | EHOD: NMG OSS |  |
|  | 1.2 | Fabian Leite | Principal Specialist: NMG Services Design |  |
|  | 1.2 | Lester Grovers | Manager: NMG OSS: Planning, Config and Inventory |  |
|  | 2.2 | Marika du Plessis | Senior Specialist: Transmission Process, Planning & Configuration Management |  |

# Distribution

| Date | Version | Name | Designation | RACI |
| --- | --- | --- | --- | --- |
| 2014-08-25 | 1.2 | Naming Convention Stakeholders Group | Various |  |

| RACI | Description |
| --- | --- |
| Responsible | Those who do work to achieve the task. The role of Responsible includes support, which is to provide resources to complete the task. (such as the author) |
| Accountable | Also Approver or final Approving authority Those who are ultimately accountable to the correct and thorough completion of the task. Accountable is the one to whom “R(s)” are accountable. In other words, A must sign off (Approve) on work that R provides. There must be only one A specified for each task. The role of Accountable may include Responsible. In other words, it is not unusual that the one who is Accountable for a task is also Responsible to do the work to achieve the task. |
| Consulted | Those whose opinions are sought. Two-way communication. (may include reviewer) |
| Informed | Those who are kept up up-to-date on progress. One-way communication. |

DOCUMENT CONTROL

This document is controlled in accordance with the procedures indicated in document ___________”.

# CONTROLLED DISTRIBUTION LIST

Once updated, a copy of the latest revision will be saved in the document management system in use.  An e-mail to this effect will be sent to the relevant personnel or heads of department.

| Copy No. | Owner | Location |
| --- | --- | --- |

# DOCUMENT CHANGE HISTORY

| ISSUE NO | DATE ISSUED | ISSUED BY | CHANGE DESCRIPTION |
| --- | --- | --- | --- |
| Draft | 08-Apr-2013 | TCS | Draft Version |
| Final | 29-Apr-2013 | TCS | OSP part updated and final version released |
| Minor Update | 29-May-2014 | Vodacom | Added section 1.2 |
| Additional Updates | 17-Jul-2014 | Vodacom | Updated all Figures in document with new diagrams and tables |
| Additional Updates | 8 to 21-Aug-2014 | Vodacom | Added Ducts and Trenches, CDN equipment, WiFi information etc |
| Final Updates | 25 Aug 2014 | Vodacom | Final Format Checks. Added additional Node functionalities in Table, added additional Nodes and Codes |
| Phase 2 Updates | 10 Oct – 25 Nov 2014 | Vodacom | Update diagrams Update CDN Information Include Labelling Products and Power Labels specification Include address under Location attributes, Appendix update of CDN nodes |
| Additional Updates | 20 Mar – 10 Apr 2015 | Vodacom | Updated new Coriant Node codes, ODF Port, Device Port and Patch Cord Naming |

# ABBREVIATIONS, ACRONYMS AND DEFINITIONS

| ABBREVIATIONS | DESCRIPTION |
| --- | --- |
| 3G | 3rd generation |
| AGR | Aggregation Layer |
| ALU | Alcatel-Lucent |
| AP | Access Point (Wi-Fi) |
| API | Application Programming Interface |
| ATM | Asynchronous Transfer Mode |
| BSS | Business Support Systems |
| BTS | Base transceiver station |
| BW | Bandwidth |
| CCMDB | Change and Configuration Management Database |
| CDMA | Code division multiple access |
| CDN | Content Delivery Network |
| CORBA | Common Object Request Broker Architecture |
| CoS | Class of Service |
| COTS | Commercial off-the-shelf |
| DWDM | Dense wavelength division multiplexing |
| E2E | End to End |
| eTOM | enhanced Telecom Operations Map |
| FAQ | Frequently asked questions |
| GSM | Global System for Mobile Communications |
| GTM | Go To Market |
| GUI | Graphical User Interface |
| Handhole | See Manhole |
| HLR | home location register |
| HP | Hewlett Packard |
| HPSD | HP Service Desk |
| IMS | IP Multimedia Subsystem |
| IMS | Inventory Management System |
| IN | Intelligent Network |
| IP | Internet Protocol |
| ISP | Inside Plant (cabling and equipment installed in a telecom operators facility) |
| IT | Information Technology |
| ITU-T | International Telecommunication Union –Telecommunication Standardization Sector |
| KPIs | Key Performance Indicators |
| L2 | Layer 2 |
| L3 | Layer 3 |
| L4 | Layer 4 |
| LTE | Long Term Evolution |
| Manhole | In this document the term Manhole is used to refer to either Manholes & Handholes |
| MTTR | Mean time to repair |
| N/W | Network |
| NE | Network Element |
| NERA | National Employment Rights Authority |
| NGN | Next-generation network |
| NGOSS | New Generation Operations Systems and Software |
| NMC | Network Management Centre |
| NMS/EMS | Network Management System/ Element Management System |
| NOC | Network Operations Center |
| NSN | Nokia Siemens Networks |
| O&M | Operations & Maintenance |
| OCM | Organizational Change Management |
| ODF | Optical Distribution Frame |
| OSP | Outside Plant (physical cabling and supporting infrastructure such as conduit, tower, manholes, etc. between two facilities) |
| RAN | Radio Access Network |
| SDH | Synchronous Digital Hierarchy |
| SDK | Software Development Kit |
| SDLC | Systems Development Life cycle |
| SI | Systems Integration |
| SIA | Service Impact Analysis |
| SID | Shared Information Data Model |
| SIT | System Integration Testing |
| SLA | Service-level agreement |
| SMBs | Small and Medium Businesses |
| SMEs | Subject Matter Experts |
| SMS | Short Message Service |
| SNMP | Simple Network Management Protocol |
| SO | Strategic Outsourcing |
| SOA | Service Oriented Architecture |
| SOAP | Simple Object Access Protocol |
| SOC | Service Operations Center |
| STM | Synchronous Transport Module |
| TCS | Tata Consultancy Services |
| uCMDB | Universal Configuration Management Database |
| VC | Virtual Container |
| VLAN | Virtual Local Area Networks |
| WiFi | Local Area Wireless Technology |
| WiMAX | Worldwide Interoperability for Microwave Access |

# RELATED DOCUMENTS

| Document number and title | Publishing Organization | Version | Date | LOCATION |
| --- | --- | --- | --- | --- |
| OSM Quickwins SOW | TCS | V 2.3 |  | MS Sharepoint |
| Analysis Document-Naming Convention_V 1.4 | TCS | V 1.4 |  | MS Sharepoint |
| Iterative Design Workshop – Initiative A V1.3 | TCS | V 1.3 |  | MS Sharepoint |

# Table of Contents

1	Introduction	15

1.1	Scope	15

1.2	Benefits	15

1.2.1	Central & Regional Network Planning Teams	15

1.2.2	Network operations centre	15

1.3	Objective	16

2	Design	18

2.1	Power Grid Area	21

2.1.1	Power Grid Area Hierarchy	21

2.1.2	Power Grid Area Object	21

2.2	Location	22

2.2.1	Location Hierarchy	22

2.2.2	Location Object	22

2.2.3	Location Attributes	23

2.3	Node	25

2.3.1	Node Naming Convention	25

2.3.2	Node Naming Components	26

2.3.3	Node Attributes	27

2.4	Link (Physical Link)	28

2.4.1	Link Naming Convention	28

2.4.2	Link Naming Components	28

2.4.3	Link Attributes	29

2.5	Circuit (Logical Link)	31

2.5.1	Circuit Naming Convention	31

2.5.2	Circuit Naming Components	31

2.5.3	Circuit Attributes	47

2.6	Cell	49

2.6.1	Cell Naming Convention	49

2.6.1	Cell Attributes	49

2.7	RACK	50

2.7.1	Rack Naming Convention	50

2.7.2	Rack Naming Components	50

2.7.3	Rack Attributes	51

2.8	ODF / Patch Panel	52

2.8.1	ODF / Patch Panel Naming Convention	52

2.8.2	ODF / Patch Panel Naming Components	52

2.8.3	ODF / Patch Panel Attributes	53

2.9	ODF / Patch Panel Port	54

2.9.1	ODF / Patch Panel Port Naming Convention	54

2.9.2	ODF / Patch Panel Port Naming Components	54

2.9.3	ODF / Patch Panel Port Attributes	54

2.10	Device Port (For ISP)	55

2.10.1	Device Port Naming Convention	55

2.10.2	Device Port Naming Components	55

2.10.3	Device Port Attributes	55

2.11	Patch Cord	56

2.11.1	Patch Cord Naming Convention	56

2.11.2	Patch Cord Naming Components	56

2.11.3	Patch Cord Attributes	57

2.12	Manhole	57

2.12.1	Manhole Naming Convention	57

2.12.2	Manhole Naming Components	57

2.12.3	Manhole Attributes	58

2.13	Pole for Overhead Fibre	59

2.13.1	Pole Naming Convention	59

2.13.2	Pole Naming Components	59

2.13.3	Pole Attributes	60

2.14	Splice Closure	60

2.14.1	Splice Closure Naming Convention	60

2.14.2	Splice Closure Naming Components	62

2.14.3	Splice Closure Attributes	62

2.15	Duct	63

2.15.1	Duct Naming Convention	63

2.15.2	Duct Naming Components	63

2.15.3	Duct Attributes	64

2.16	Trench	65

2.16.1	Trench Route Naming Convention	65

2.16.2	Trench Route Naming Components	65

2.16.3	Trench Attributes	66

2.16.4	Trench Sector Naming Convention	67

2.16.5	Trench Sector Naming Components	68

2.17	Cable	68

2.17.1	Cable Naming Convention	68

2.17.2	Cable Naming Components	69

2.17.3	Cable Attributes	70

3	References	72

3.1	Naming Convention Tool	72

4	Appendix	73

4.1	Country Code	73

4.2	ZA Province Code	73

4.3	LS Province (District) Code	73

4.4	Power Grid Area	74

4.5	Location Code Generation Algorithm	74

4.6	Type of site	76

4.7	Site Owner	77

4.8	Vodacom Operating Region	77

4.9	POC Type	77

4.10	Node / Device Type Code	78

4.11	Business Area / Internal Customer	83

4.12	Node Status	83

4.13	Node Functionality	85

4.14	Link Medium	86

4.15	Link Bandwidth and Bandwidth Code	86

4.16	Protection Type	87

4.17	Link Operator Type	87

4.18	Link Status	87

4.19	Circuit Function Code	88

4.20	Bandwidth to Value Mapping	90

4.21	Circuit Status	91

4.22	Circuit Service Type	91

4.23	Rack Type Code List	92

4.24	ODF / Patch Panel Type Code List	92

4.25	Cable Type Code List	92

4.26	Cable Manufacturer List	93

4.27	Civil works/Fibre Build Contractor List	93

4.28	Fibre Size Code List	93

4.29	Network Type Code List	94

4.30	Cable Owner List	94

4.31	Cable Request Type	94

4.32	Duct Type	95

4.33	Rack Owner	95

4.34	Manhole Type Attribute List	95

4.35	Pole Manufacturer and Specification	96

4.36	Vodacom Counties	96

4.37	Physical Trunk Types	97

4.38	Broadband Interface Types	98

4.39	Product Specification for Cable Labels	98

4.40	Power Components to be labelled:	99

4.41	Vodacom Manhole and/or Pole Label Specification:	100

Table List

Table 1 Object List	16

Table 2 Object List out of scope	17

Table 3 Vodacom Objects mapped to Standard Object Type	20

Table 4 Power Grid Type Relationships	21

Table 5 Location Type Relationships	23

Table 5 Location Attributes	24

Table 6 Node Naming Components	26

Table 7 Node Attributes	27

Table 8 Link Naming Components	28

Table 9 Link Attributes	30

Table 10 Circuit Naming Components	31

Table 11 Circuit Types for SDH& PDH Transmission & CDN	33

Table 12 Circuit Types for IP Transmission & CDN	37

Table 13 NSN 3G RAN Access Logical Links	39

Table 14 Huawei 3G RAN Access Logical Links	40

Table 15 NSN 2G RAN Core Logical Links	41

Table 16 NSN 3G RAN Core Logical Links	42

Table 17 Huawei 2G RAN Core Logical Links	42

Table 18 Huawei 3G RAN Core Logical Links	43

Table 19 Circuit Types and Circuit Type Code for Mobile Core Logical Links	44

Table 20 Circuit Attributes	47

Table 21 Rack Naming Components	50

Table 22 Rack Attributes	51

Table 23 ODF / Patch Panel Naming Components	52

Table 24 ODF / Patch Panel Attributes	53

Table 25 ODF / Patch Panel Port Naming Components	54

Table 24 ODF / Patch Panel Port Attributes	54

Table 26 Device Port Naming Components	55

Table 24 ODF Port Attributes	55

Table 27 Patch Cord Naming Components	56

Table 30 Manhole Naming Components	58

Table 31 Rack Attributes	58

Table 32 Manhole Naming Components	59

Table 29 Rack Attributes	60

Table 30 Splice Closure Naming Components	62

Table 31 Splice Closure Attributes	62

Table 32 Cable Naming Components	63

Table 33 Splice Closure Attributes	64

Table 34 Trench Naming Components	65

Table 35 Splice Closure Attributes	66

Table 34 Trench Naming Components	68

Table 36 Cable Naming Components	69

Table 37 Splice Closure Attributes	70

Table 43 Country Code	 _Country_Attributes	73

Table 44 Province Code	 _Province_Attributes	73

Table 44 Province Code	 _ LS Province (District)_Attributes	73

Table 44 Power Grid Areas	74

Table 45 Type of Site	76

Table 46 Site Owner	77

Table 47 Operating Region	77

Table 43 POC Type for Site	77

Table 49 Device Type Code	83

Table 50 Business Area / Internal Customer	83

Table 46 Node Status	84

Table 53 Node Functionality	85

Table 54 Link Type and Link Type Code	86

Table 55 Link Bandwidth and Bandwidth Code	86

Table 56 Protection Type	87

Table 57 Link Operator Type	87

Table 58 Link Status	87

Table 59 Circuit Function Code	90

Table 60 Bandwidth to Value Mapping	91

Table 61 Circuit Status	91

Table 62 Circuit Service Type	91

Table 63 Rack Type List	92

Table 64 ODF / Patch Panel Type List	92

Table 65 Cable Type List	92

Table 66 Cable Manufacturer List	93

Table 67 Cable Supplier List	93

Table 68 Fibre Size List	93

Table 69 Network Type List	94

Table 70 Cable Owner List	94

Table 71 Cable Request Type	95

Table 72 Duct Type List	95

Table 73 Rack Owner	95

Table 74 Manhole Type List	96

Table 75 Pole Manufacturer and Specification List	96

Table 76 Vodacom Counties	97

Table 77 Trunk Types	98

Table 78 Broadband Interface Types	98

Table 79 Product Specification for Cable Labels	99

Table 80 Broadband Interface Types	100

Table 82 Vodacom Manhole or Pole Label Specification	100

Table of Figure

Figure 1 Power Grid Area Hierarchy	21

Figure 2 Location Hierarchy	22

Figure 2 Node Naming Convention	26

Figure 3 Link Naming Convention	28

Figure 4 Circuit Naming Convention	31

Figure 5 Tellabs Network Architecture	34

Figure 6 PDH Circuit Hierarchy with STM between 6300 and 8100	34

Figure 7 PDH Circuit Hierarchy with E1 between 6300 and 8100	35

Figure 8 PDH Circuit Hierarchy with Concatenation of VC12’s	35

Figure 9 VC4 concatenation scenarios	35

Figure 10 EoS Scenario	36

Figure 11 SDH Circuit Hierarchy	36

Figure 12 SDH Circuit Hierarchy with Concatenation of TU12, TU3 and AU4	36

Figure 13 VPN Circuit Hierarchy back	37

Figure 14 Ethernet Ring	38

Figure 15 NSN 2G RAN Access Logical Links	39

Figure 16 NSN 3G IuB Hierarchy	39

Figure 17 Huawei 2G RAN Access Logical Links	40

Figure 18 Huawei 3G IuB Hierarchy	41

Figure 19 NSN 2G RAN Core Circuit Hierarchy	41

Figure 20 NSN 3G RAN Core Circuit Hierarchy	42

Figure 21 Huawei 2G RAN Core Circuit Hierarchy	43

Figure 22 Huawei 3G RAN Core Circuit Hierarchy	44

Figure 23 STP Links	45

Figure 24 Signalling and Traffic between MSC and BSC	45

Figure 25 Signalling and Traffic between MSS and MGW	46

Figure 26 Trunk Groups from MSCs to other Nodes	46

Figure 27 Cell Naming Convention	49

Figure 28 Rack Naming Convention	50

Figure 29 ODF/ Patch Panel Naming Convention	52

Figure 30 ODF / Patch Panel  Port	54

Figure 32 Device Port Naming Convention	55

Figure 34 Manhole Naming Convention	57

Figure 35 Duct Naming Convention	63

Figure 36 Trench Route Naming	65

Figure 37 Trench Sector Naming Convention	67

Figure 38 Cable Naming Convention	68

# Introduction

## Scope

- Implement a Standard Naming Convention across the different domains; Transport, RAN, and Core.  Most of the IP domain will be included in the next phase of the Standard Naming Convention project.

- This includes end-to-end process across the different domains to fill the existing gaps and ensure consistent use of the Naming Convention

## Benefits

### Central & Regional Network Planning Teams

- Efficient and proficient planning of services which avoid order rejections and rework

- Improve inventory keeping and management for capacity and traffic management over the circuits

- Easy identification of resources during planning

- Can be easily extended to any new vendor or technology in the future

### Network operations centre

- Improved MTTR and location of faults

- Will enable pro-active monitoring of faults due to unification of names within network entities

- Will enable efficient service to Network Correlation and Service Impact Analysis in the case of any fault. This will help ensure that the customer is proactively informed.

## Objective

The objective of the Initiative is to define a Standard Naming Convention for ISP (Active) and OSP (Passive) objects. The following table gives the list of objects currently named in Vodacom for active and passive equipment.

| Technology | Object |
| --- | --- |
| Transmission | Site |
|  | Node |
|  | Link |
|  | Trunk |
|  | Tunnel |
|  | Pseudowire |
|  | IP MPLS Trunk |
|  | Service Circuit |
| RAN | Site |
|  | BTS |
|  | BSC |
|  | RNC |
|  | NodeB (add LTE stuff) |
|  | Cell |
| Core | Site |
|  | HLR |
|  | VLR |
|  | GGSN |
|  | SGSN |
|  | STP |
|  | MSS |
|  | MGW (Add LTE stuff) |
|  | Power Grid Area |
| OSP | Site |
|  | Rack |
|  | Ducts |
|  | ODF |
|  | Cable |
|  | Manhole |
|  | Patch Panel |
|  | Splice Tray |
| CDN | Site |
|  | Nodes |
|  | Links |
|  | Trunk |
|  | Tunnel |
|  | IP MPLS Trunk |
|  | Service Circuit |

Table 1 Object List

List of objects for which the design will be covered in next phases.

| Technology | Object |
| --- | --- |
| ISP | Rectifiers |
|  | Batteries |
|  | Generators |
|  | Fire Suppression |
|  | Extinguishers |
|  | Detectors |
|  | Air Conditioners |
|  | Power Supply Authority |
|  | Power Meter |
|  | Site Controller |
|  | Alarm/Security |
|  | Earth Cables |
|  | Earth Bars/Spikes |
|  | Lightning Protection |
|  | Manholes |
|  | Masts/Towers |

Table 2 Object List out of scope

# Design

The table below gives the definition of the Standard Object Types considered for defining Naming Convention

The following section explains the mapping of the Standard Object Types with Vodacom Network entities.

| Technology | Object | Mapped Standard Object Type |
| --- | --- | --- |
| Transmission | Site | Location |
|  | Node | Node |
|  | Trunk | Link |
|  | Tunnel | Circuit |
|  | Pseudowire | Circuit |
|  | IP MPLS Trunk | Circuit |
|  | Service Circuit | Circuit |
| RAN | Site | Location |
|  | BTS | Node |
|  | BSC | Node |
|  | RNC | Node |
|  | NodeB | Node |
|  | Cell | Cell |
| Core | Site | Location |
|  | HLR | Node |
|  | VLR | Node |
|  | GGSN | Node |
|  | SGSN | Node |
|  | STP | Node |
|  | MSS | Node |
|  | MGW | Node |
| OSP | Site | Location |
|  | Rack | Rack |
|  | Sub Rack | Node |
|  | ODF | ODF |
|  | Pole | Pole |
|  | Splice Closure | Splice Closure |
|  | Duct | Duct |
|  | Trench | Trench |
|  | Patch Cord | Patch Cord |
|  | Cable | Cable |
|  | Manhole | Manhole |
|  | Patch Panel | Patch Panel |
| CDN | Site | Site |
|  | Node | Node |
|  | Link | Link |
|  | Trunk | Link and Circuit |
|  | Tunnel | Circuit |
|  | IP MPLS Trunk | Circuit |
|  | Service Circuit | Circuit |

Table 3 Vodacom Objects mapped to Standard Object Type

## Power Grid Area

### Power Grid Area Hierarchy

Figure 1 shows the Proposed Power Grid Hierarchy. The new Power Grid hierarchy is based on South Africa’s geographic boundaries starting from Country, Province (9 Geographical Provinces), and then the Power Grid Area Name.

Country will be the highest level for the Power Grid Area Object. Province will be the next level of Power Grid Area Object and will be created under a country when the user wants to add a new Province using the Naming Convention Tool.  The Power Grid Areas of Eskom are mapped according to the 9 Geographical Provinces and this is less likely to change.

The last level of Power Grid Area Object will be a Power Grid Area Name. The Power Grid Area can be mapped to Vodacom Locations (sites) which contains attributes such as Vodacom Operational Region and County for Fault Management.  Short Codes will be generated automatically for all Location Objects including Site when the Location Object is getting created using the Naming Convention Tool (Refer section 3.1 Naming Convention Tool)

Figure 1 Power Grid Area Hierarchy

### Power Grid Area Object

Following table shows the Power Grid Area type relationships with example.

Table 4 Power Grid Type Relationships

## Location

### Location Hierarchy

Figure 2 shows the Proposed Location Hierarchy. The new location hierarchy is based on South Africa’s geographic boundaries starting from Country, Province (9 Geographical Provinces), and Location.

Country will be the highest level Location Object. Province will be the next level of Location Object and will be created under a country when the user wants to add a new Province using the Naming Convention Tool.

The last level of Location Object will be a Site which will be created under a building. Short Codes will be generated automatically for all Location Objects including Site when the Location Object is getting created using the Naming Convention Tool (Refer section 3.1 Naming Convention Tool)

Figure 2 Location Hierarchy

### Location Object

Following table shows the location type relationships with example.

Table 5 Location Type Relationships

### Location Attributes

Following table gives the attributes for the Location Objects.

| Location Entity | Attribute Name | Attribute Type | Example |
| --- | --- | --- | --- |
| Country | Country Code | Automatic | ZA for South Africa Refer section 4.1 Country Code for the list of all countries with their code |
| Province | Province Code | Automatic | W for Western Province Refer section 4.2 Province Code |
| Location | Location Name | Mandatory | The location name will be imported from tools being used in the existing process (e.g. Site Name from Atoll) |
|  | Location Code | Automatic | TECCE for Techno Centre (Location code will use the Location Name & apply an algorithm to determine the location code) Refer section 4.3 Location Code Generation Algorithm |
| Location Address | Street Number | Optional | 23 |
|  | Street Name | Mandatory | Vodacom Boulevard |
|  | Building Number | Optional | 01 |
|  | Building /Farm Name/Settlement | Mandatory | Vodacom Corporate Park |
|  | Suburb / Subplace/ District | Mandatory | Noordwyk |
|  | City / Town | Mandatory | Midrand |
| Location Co-ordinates | Latitude | Mandatory | -26.15072222 |
|  | Longitude | Mandatory | 28.03397222 |
| Other Location Detail | Operating Region | Automatic | Operation region to be determined based on location (LAT/LON), e.g. WES Refer section 4.6 Vodacom Operating Region |
|  | County | Automatic | County to be determined based on location (LAT/LON), e.g.WES_RC Refer section Table 67 POC Type |
|  | Site Owner | Mandatory | Vodacom Refer section 4.5 Site Owner |
|  | BS Number | Automatic | 0022679 |
|  | Atoll ID | Automatic for RAN Site | SGA_8087 |
|  | POC Type | Mandatory – Manually for interim | POC1 Refer section 4.7 POC Type |
|  | Floor | Optional | 1,2,3,4 etc |
|  | Sub floor | Optional | 1a, 1b etc |
|  | Room | Optional | NIS Meeting Room |
|  | Type of Site | Mandatory |  |

Table 5 Location Attributes

#### Location Attributes

The following section explains the attributes on the location object.

##### Street Number

This attribute captures the number of the street according to cadastral information. This is not a mandatory field, due to some of our locations which are in rural areas

The Street number will be imported from tools used in the existing process flow (e.g. TxMan).

##### Street Name

##### This attribute captures the street name where the Location is located. This is a mandatory field and should be captured as far as possible.

##### The Street name will be imported from tools used in the existing process flow (e.g. TxMan).

##### Building Number

This attribute captures the number of the building for urban areas. This is not a mandatory field, due to some of our locations situated in rural areas.

The building number will be imported from tools used in the existing process flow (e.g. TxMan).

##### Building Name/Farm Name/ Settlement

This attribute captures the Building Name for urban areas and in case of rural areas, this attribute will capture the farm names.

The building name will be imported from tools used in the existing process flow (e.g. TxMan).

##### Suburb / Subplace/ District

This attribute captures the suburb or subplace or distict of the location.

The Suburb / Subplace will be imported from tools used in the existing process flow (e.g. TxMan).

##### City / Town

This attribute captures the City or Town of the location.

The City / Town will be imported from tools used in the existing process flow (e.g. TxMan).

##### Latitude

This attribute captures the geographical latitude of the location.

The Latitude will be imported from tools used in the existing process flow (e.g. Atoll). Decimal format will be used, e.g. -26.15072222

##### Longitude

This attribute captures the geographical longitude of the location.

The Longitude will be imported from tools used in the existing process flow (e.g. Atoll). Decimal format will be used, e.g. 28.03397222

##### Operating Region

This attribute will capture the Operating Region for a site. The attribute will be automatically determined based on the geographical location information (LAT/LON) of the location.  Refer section 4.6 Vodacom Operating Region

##### County

This attribute will contain the County for a site. The attribute will be automatically determined & imported based on the geographical information (LAT/LON) of the location. Refer section 4.6 Vodacom Operating Region

##### Site Owner

This attribute will capture as to who owns the site i.e. if the site is owned by Vodacom, Telkom, MTN, Cell C, Neotel, etc. The attribute will be imported from either Atoll or TX Manager depending of the type of site. Refer section 4.5 Site Owner

##### BS Number

This attribute will capture the BS number of a site generated from Atoll for RAN Site and TX SOM for rest of the sites. The attribute will imported as part of the site creation process.

##### Atoll ID

This attribute will capture the Atoll ID of a site generated from Atoll. The attribute will be imported as part of the site creation process. It will be null for other sites apart from RAN sites.

##### POC Type

This attribute will POC Type for the site. The attribute will be a drop-down which the user will select while updating a site using Connect Master. Consideration is currently being given to automate the POC type based on existing algorithms.

Refer section 4.7 POC Type

##### Floor

This attribute will capture the floor number of the location if the location is based in a building. The attribute will be a free text box which the user will populate while updating a site using ConnectMaster. Spaces will not be allowed.

For e.g.: Floor-1

##### Sub-Floor

This attribute will capture the sub-floor number of the location if the location is based in a building. The attribute will be a free text box which the user will populate while updating a site using ConnectMaster. Spaces will not be allowed.

For e.g.: Floor_1A

##### Room

This attribute will capture the room of the location if the location is based in a building. The attribute will be a free text box which the user will populate while updating a site using ConnectMaster. Spaces will not be allowed.

For e.g.: NMC_Room_Techno_Centre_Building

##### Type of Site

The attribute will be a dropdown which the user will select while creating a site under a building using the naming convention tool. Refer section 4.4 Type of site

## Node

### Node Naming Convention

Below is the proposed naming convention for Node.

Figure 2 Node Naming Convention

### Node Naming Components

The following table gives the components of the new naming convention.

| Node Naming Components | No of Characters | Example |
| --- | --- | --- |
| Country Code | 2 Characters | ZA |
| Region Code | 1 Character | W |
| Location Code | 5 Characters | TECHC |
| Device Type Code | 6 Characters | TL8605 Refer section 4.8 Node / Device Type Code |
| Instance Number | 2 digits | 01 |

Table 6 Node Naming Components

#### Location Component

User will first select the site where the node is to be placed.

The Site will be created as per Table 3 Location Type Relationships

The Country Code, Province Code and the Location Code will be automatically derived from the location name and location’s attributes.

#### Device Type Code Component

The list of Model type as per vendor selected, will be available as a drop down which the user will select using the Naming Convention tool. Refer section 4.8 Node / Device Type Code. Type Code will be automatically generated by the tool while adding the new types as a part of metadata creation. List in the appendix section if for example.

#### Instance Number

The Instance Number will be calculated automatically. The instance number will be incremented per node under a particular location based on the vendor and model of the device.

### Node Attributes

Based on the requirements and data analysis, there are certain components which are captured part of name but can be derived from a relationship or attribute value.

Following table gives the attributes which are required to be filled while generating the new Node name.

| Attribute Name | Attribute Type | Character Length | Example |
| --- | --- | --- | --- |
| Business Area / Internal Customer | Mandatory | Enumeration | Enterprise Business Unit Refer section |
| Status | Mandatory | Enumeration | In Service Refer section 4.10 Node Status |
| Parent Node (This will be applicable to BTS,BSC, RNC and NodeB) | Mandatory for specific Nodes | 16 Characters | ZAGMIDRNHW390001 |
| Functionality | Mandatory | Enumeration | BTS (Add CDN’s) Refer section 4.11 Node Functionality |
| Protection Type | NA | NA | This will be covered when IMS is implemented. Refer section 4.14 Protection Type |
| Bay | Optional |  | A:02 |

Table 7 Node Attributes

#### Business Area / Internal Customer Attribute

Different naming conventions are followed in Tellabs node as per the Business Area / Internal Customers. The same will be captured as part of the Business Area / Internal Customer attribute for nodes.

Business Area / Internal Customer will appear as a drop down to users when they are generating node name using the Naming Convention Tool. Business Area / Internal Customer

#### Status Attribute

The current RAN Naming Convention contains the status of the node in the name.

For eg: When the node is planned the BTS name contains “NEW”. This component will be stored as an attribute to the node. The attribute will be a dropdown which the user will select while generating the node name using the Naming Convention Tool. Refer section 4.10 Node Status for the list of statuses

#### Parent Node Attribute

In existing RAN Naming Convention, the BTS name contains the parenting BSC area name and the BSC name contains the parenting MSC name, therefore this attribute will be applicable for RAN nodes only. The attribute will be a drop-down box where users will select parenting node name while updating the BTS/ BSC/ NodeB/ RNC name using Naming Convention Tool.

#### Functionality Attribute

Technology related information like 2G/3G or LTE are currently being captured in name of RAN Node. This attribute will be provided for users to capture this information. BTS value is equivalent to 2G, NodeB is equivalent to 3G and eNodeB is equivalent to LTE. This will be applicable for RAN nodes and the Mobile Core nodes. The attribute will be a drop down and the user will select the value while generating the node name using the Naming Convention Tool. Refer section 4.11 Node Functionality

## Link (Physical Link)

### Link Naming Convention

Below is the proposed naming convention for Link

Figure 3 Link Naming Convention

### Link Naming Components

The following table gives the Link Naming Components

| Link Naming Components | No of Characters | Example |
| --- | --- | --- |
| A-end terminating node | 16 Characters | ZAWTECHCTL860501 |
| Z-end terminating node | 16 Characters | ZAWCENTYTL860501 |
| Link Medium | 3 Characters | FIB Refer section 4.12 Link |
| Instance Number | 2 Digits | 01 |

Table 8 Link Naming Components

#### A-End and Z-End Terminating Nodes Component

User will select the A-end and Z-end sites while generating the link name using the Naming Convention Tool. All the nodes at the selected A-end and Z-end sites will be displayed; user will select the appropriate node at the site.

#### Link Medium Component

The Link Medium Type will show the medium used between the two nodes. The Link Medium will be a dropdown which the user will select while generating the link name using the Naming Convention Tool. A 3 Character code will be generated for each Link Type. Refer section 4.12 Link  for link medium identified with their link codes. Type Code will be automatically generated by the tool while adding the new types as a part of metadata creation

#### Instance Number Component

The Instance Number will be calculated automatically. The instance number will be incremented per link between two nodes per link type.

### Link Attributes

Based on the requirements and data analysis, there are certain components which are captured part of name but can be derived from a relationship or attribute value.

Below are such identified attributes which are required to be filled while generating the new Link name.

| Attribute Name | Attribute Type | Character Length | Example |
| --- | --- | --- | --- |
| Bandwidth | Mandatory | 5 Characters | 100Mx Refer section 4.13 Link Bandwidth and Bandwidth Code |
| Protection Type | Mandatory | Enumeration | Primary Refer section 4.14 Protection Type |
| Backup Link Name | Mandatory if “Secondary” protection type is selected | 50 Characters | ZAWTECHCTL860501_ ZAWCENTYTL860501_FIB_10Gxxx_002 |
| Status | Mandatory | Enumeration | In Service Refer section 4.16 Link Status |
| Operator Type | Mandatory | Enumeration | Telkom Refer section 4.15 Link Operator Type |
| Business Area | Mandatory | Enumeration | Vodacom Business Refer Section Business Area / Internal Customer |
| Link Number | Optional | 50 Characters | EML0001290 |
| Physical Trunk Type | NA | NA | Ethernet Refer section 4.31 This will be included once IMS is in place |
| Broadband Interface | NA | NA | Electrical Refer section 4.32 Broadband Interface Type This will be included once IMS is in place |

Table 9 Link Attributes

#### Bandwidth Component

The Bandwidth component will show the bandwidth of the link. User will have an option of selecting either from the Bandwidth available in the dropdown or define a new bandwidth if the required bandwidth value is not present in the list while population the link attributes.  Refer section 4.13 Link Bandwidth and Bandwidth Code for the indicative list of bandwidth values currently configured in Vodacom with their bandwidth codes.

#### Protection Type Attribute

The current Trunk Naming Convention captures whether the Trunk is Primary or Secondary in the Trunk Name. This will now be captured as an attribute to the link. The attribute will be a dropdown which the user will select while generating the link name using the Naming Convention Tool. Refer section 4.14 Protection Type for the Protection values for a Link.

#### Backup Link Name Attribute

Backup Link Name is required to identify the Primary Secondary link pair. When the Primary link name is generated using the Naming Convention tool, the Secondary link name will be populated in this field and vice versa. The attribute will be supported by a search box which will allow the user to find the backup link.

#### Status Attribute

Status will be stored as an attribute to the Link. The attribute will be a dropdown which the user will select while generating the Link name using the Naming Convention Tool. Refer section 4.16 Link Status

#### Operator Type Attribute

Operator Type will be a dropdown which the user will select while generating the Link name using the Naming Convention Tool. Refer section 4.15 Link Operator Type for list of Operator Types

#### Link Number Attribute

The current Trunk Naming Convention captures the link number generated in TX Manager in the Trunk Name. This will now be captured as an attribute to the link. Link Number will be passed from TX SOM in case of links been requested by TX SOM. In other cases, users will need to fill the Link Numbers manually.

#### Business Area Attribute

Business Area like VB is captured in the current name. This will now be captured as an attribute to the link. The attribute will be a dropdown which the user will select while generating the link name using the Tool.

## Circuit (Logical Link)

### Circuit Naming Convention

Below is the proposed naming convention for Circuit.

Figure 4 Circuit Naming Convention

### Circuit Naming Components

The following table gives the Circuit Naming Components

| Link Naming Components | No of Characters | Example |
| --- | --- | --- |
| A-end terminating node | 16 Characters | ZAWTECHCTL860501 |
| Z-end terminating node | 16 Characters | ZAWCENTYTL860501 |
| Circuit Function Code | 8 Characters | VC12xxxx Refer section 4.17 Circuit Function Code |
| Instance Number | 3 digits | 001 |

Table 10 Circuit Naming Components

#### Terminating Nodes Component

User will select the A-end and Z-end sites while generating Circuit name using Naming Convention Tool. All the nodes at the selected A-end and Z-end sites will be displayed; user will select the appropriate node at the site.

#### Circuit Function Code Component

The Circuit Type will be a component of the Circuit Name. User will select the Circuit Type and corresponding bandwidth type using the Naming Convention Tool. The tool should automatically generate the Circuit Function Code based on the user selection. The Circuit Function Code comprised of two components:

- Circuit Type

- Bandwidth Value (in kbps, mbps or gbps) which is based on user selection of bandwidth and the multiplier value. Each bandwidth type has a corresponding bandwidth value as per standards.

When users select a circuit type where there is no concatenation scenario (no multiplier) defined in the document, tool should not ask for multiplier to users and use the pre-defined function code.  Refer the circuit function code table for examples.

The following below sub-sections will describe the various Circuit Types in Transmission, RAN and Mobile core identified from Vodacom Network mapped with Standard Technology and Circuit Types. Type Code will be loaded as prescribed in the document for the existing Circuit Types. Users should be able to define circuit function code while adding the new types as a part of metadata creation

##### Transmission

Circuits (Logical Link) Type

1/0 VC-12 Trunk

1/0 VC-12-mc Trunk

1/0 VC-2-mc Trunk

4/1 VC4 Trunk

4/1 VC4 Trunk(Ethernet)

4/1 VC-4 External Trunk

1/0 8M Trunk

OCH Trunk

SDH tu12 Circuit

SDH tu12 Circuit (Ethernet)

SDH tu3 Circuit

SDH tu3 Circuit (Ethernet)

SDH tu2 Circuit

SDH tu2 Circuit (Ethernet)

SDH AU4 Circuit

SDH AU4 Circuit (Ethernet)

PDH 1/0 Circuit

E-LAN

VLAN

VLAN 802.1Q

VRF (MPLS)

Tunnels

E1 Circuit

P2P Circuit

EVC

EPL

EVPL

PPP Trunk

Frame Relay Trunk

Frame Relay Virtual Circuit

Ethernet

Table 11 Circuit Types for SDH& PDH Transmission & CDN

Below are the various network scenarios identified in Vodacom Transmission Network.

Transmission Circuit Hierarchy

The below diagram is the high level Vodacom South Africa Transmission Network Architecture. Transmission Core Network comprises of Tellabs 7100 DWDM Rings connected with outer Tellabs 6300 and 8800 series of devices to create an aggregate ring. Tellabs 8100 series of nodes creates PDH Ring and 8600 series of access nodes are the serving nodes in Tellabs architecture.

Figure 5 Tellabs Network Architecture

Figure 6 PDH Circuit Hierarchy with STM between 6300 and 8100

Figure 7 PDH Circuit Hierarchy with E1 between 6300 and 8100

Figure 8 PDH Circuit Hierarchy with Concatenation of VC12’s

Figure 9 VC4 concatenation scenarios

Figure 10 EoS Scenario

Figure 11 SDH Circuit Hierarchy

Figure 12 SDH Circuit Hierarchy with Concatenation of TU12, TU3 and AU4

Following table shows the Transmission Ethernet, IP and ATM Logical Circuits mapped to Standard Circuit Types

Logical Link Type

VLAN Trunk

VLAN VPN

ATM_L2VPN

Ethernet L2 VPN

TDM L2 VPN

BGPMPLSVPN

S-VLAN

Pseudowire

IP/MPLS Trunk

Tunnel

Table 12 Circuit Types for IP Transmission & CDN

Figure 13 VPN Circuit Hierarchy back

Figure 14 Ethernet Ring

##### RAN

Following table shows the logical link types in RAN Access segregated based on technology and Vendor. Each table has vendor information, Technology like 2G or 3G, Underlying Technology layer like SDH or IP, Logical Link Type, Mapped Circuit Type and Interface Type. Logical Link Type is mapped with standard Circuit Type which will be used in Naming Convention Tool.

NSN 2G

Figure 15 NSN 2G RAN Access Logical Links

NSN 3G

| Vendor | Technology | Transport Technology | Logical Link Name | Mapped Standard Circuit Type | Interface Type |
| --- | --- | --- | --- | --- | --- |
| NSN | 3G | IP | O&M DCN | O&M | IuB |
|  |  |  | IPNB | Traffic |  |
|  |  | ATM | O&M DCN | O&M |  |
|  |  |  | COCO | Traffic |  |

Table 13 NSN 3G RAN Access Logical Links

Figure 16 NSN 3G IuB Hierarchy

Huawei 2G

Figure 17 Huawei 2G RAN Access Logical Links

Huawei 3G

| Vendor | Technology | Transport Technology | Logical Link Name | Mapped Standard Circuit Type | Interface Type |
| --- | --- | --- | --- | --- | --- |
| Huawei | 3G | ATM | IPOA | O&M | IuB |
|  |  |  | AAL2PATH | Traffic |  |
|  |  |  | NCP/CCP/ALCAP | Signalling |  |
|  |  | IP | OMCH | O&M |  |
|  |  |  | IP PATH | Traffic |  |
|  |  |  | SCTP | Signalling |  |

Table 14 Huawei 3G RAN Access Logical Links

Figure 18 Huawei 3G IuB Hierarchy

##### CORE

NSN 2G

| Vendor | Technology | Transport Technology | Logical Link Name | Mapped Standard Circuit Type | Interface Type |
| --- | --- | --- | --- | --- | --- |
| NSN | 2G | SDH | ATER | ATER | ATER |
|  |  | IP | DATA | Traffic | Gb |
|  |  |  | M3UA | Signalling |  |

Table 15 NSN 2G RAN Core Logical Links

Figure 19 NSN 2G RAN Core Circuit Hierarchy

NSN 3G

| Vendor | Technology | Transport Technology | Logical Link Name | Mapped Standard Circuit Type | Interface Type |
| --- | --- | --- | --- | --- | --- |
| NSN | 3G | IP | DATA | Traffic | IUPS |
|  |  |  | M3UA | Signalling |  |
|  |  | ATM | DATA | Traffic | IUR |
|  |  |  | MTP3 | Signalling |  |
|  |  |  | DATA | Traffic | IUCS |
|  |  |  | MTP3 | Signalling |  |

Table 16 NSN 3G RAN Core Logical Links

Figure 20 NSN 3G RAN Core Circuit Hierarchy

Huawei 2G

| Vendor | Technology | Transport Technology | Logical Link Name | Mapped Standard Circuit Type | Interface Type |
| --- | --- | --- | --- | --- | --- |
| Huawei | 2G | SDH | ATER | ATER | ATER |
|  |  |  | A | A | A |
|  |  | IP | Gb | Gb | Gb |

Table 17 Huawei 2G RAN Core Logical Links

Figure 21 Huawei 2G RAN Core Circuit Hierarchy

Huawei 3G

| Vendor | Technology | Transport Technology | Logical Link Name | Mapped Standard Circuit Type | Interface Type |
| --- | --- | --- | --- | --- | --- |
| Huawei | 3G | ATM | AAL2 PATH | Traffic | IUCS |
|  |  |  | MTP3 | Signalling |  |
|  |  | IP | IP PATH | Traffic | IUPS |
|  |  |  | SCTP | Signalling |  |
|  |  | ATM | AAL2 PATH | Traffic | IUR |
|  |  |  | MTP3 | Signalling |  |

Table 18 Huawei 3G RAN Core Logical Links

Figure 22 Huawei 3G RAN Core Circuit Hierarchy

The following table shows the Mobile Core logical links.

Logical Link Name

STP

SS7

Trunk Groups

Circuit Group

Table 19 Circuit Types and Circuit Type Code for Mobile Core Logical Links

Following diagrams show the logical connectivity between the Mobile Core devices.

Figure 23 STP Links

Figure 24 Signalling and Traffic between MSC and BSC

Figure 25 Signalling and Traffic between MSS and MGW

Figure 26 Trunk Groups from MSCs to other Nodes

#### Instance Number Component

The Instance Number will be calculated automatically. The instance number will be incremented between the two nodes per circuit type.

### Circuit Attributes

Based on the requirements and data analysis, there are certain components which are captured part of name but can be derived from a relationship or attribute value.

Below are such identified attributes which are required to be filled while generating the new Circuit name.

| Attribute Name | Attribute Type | Character Length | Example |
| --- | --- | --- | --- |
| Protection Type | Mandatory | Enumeration | Primary Refer section 4.14 Protection Type |
| Backup Circuit Name | Optional, but Mandatory when “Secondary” Protection Type is selected | 50 Characters | ZAWTECHCTL860501_ZAWCENTYTL860501_VC12xxxx_004 |
| Status | Mandatory | Enumeration | Planned Refer section 4.18 Circuit Status |
| Circuit Service Type (This will be applicable to Service Circuit Only) | Optional | Enumeration | ABIS Refer section 4.19 Circuit Service Type |
| A-end service node (This will be applicable to Service Circuit Only) | Optional | 50 Characters | ZAWCENTY HWB69G01 (BSC node connecting to A-end Tellabs) |
| Z-end service node (This will be applicable to Service Circuit Only) | Optional | 50 Characters | ZAWTECHC HWB39A01 (BTS node connecting to Z-end Tellabs) |
| Business Area/ Internal Customer | Mandatory | 3 Characters | EBU etc |

Table 20 Circuit Attributes

#### Protection Type Attribute

The Protection status will be captured as an attribute to the Circuit. The attribute will be a dropdown which the user will select while generating the Circuit name using the Naming Convention Tool. Refer section 4.14 Protection Type

#### Backup Circuit Name Attribute

Backup Link Circuit is required to identify the Primary Secondary Circuit pair. When the Primary Circuit name is generated using the Naming Convention tool, the Secondary Circuit name will be populated in this field and vice versa. The attribute will be supported by a search box which will allow the user to find the backup link.

#### Status Attribute

Status will be stored as an attribute to the Circuit. The attribute will be a dropdown which the user will select while generating the Circuit name using the Naming Convention Tool. Refer section 4.18 Circuit Status

#### Circuit Service Type Attribute

The E2E Tellabs Circuit is used to provide services for RAN and Core Links i. e Signalling, Traffic and O&M. These service types were earlier captured in the circuit names. The Circuit Service Type will be an attribute to the Circuit which will be a dropdown which the user will select while generating the Circuit name using the Naming Convention Tool. Refer section 4.19 Circuit Service Type

#### Service node Attribute

The current Tellabs Circuits and the MPLS VPNs captures the BTS and BSC codes in the Circuit Name. In the new convention this will be captured as attributes to the Circuit as A-end Service Node and Z-end Service Node. The attribute will be supported by a search box which will allow the user to find the backup link.

This attribute will be applicable to the highest level Tellabs Circuit only.

#### Business Area / Internal Customer Attribute

Different naming conventions are followed in the circuit as per the Business Area / Internal Customers. The same will be captured as part of the Business Area / Internal Customer attribute for nodes.

Business Area / Internal Customer will appear as a drop down to users when they are generating node name using theTool. Refer Business Area / Internal Customer

## Cell

### Cell Naming Convention

Below is the proposed naming convention for Cell. New convention for Cell is same as current Cell Naming Convention but since the BTS/Node B naming convention has changed new Cell name will also change.

Figure 27 Cell Naming Convention

### Cell Attributes

No Attributes defined.

## RACK

### Rack Naming Convention

Below is the proposed naming convention for Rack.

Figure 28 Rack Naming Convention

### Rack Naming Components

| Naming Components | No of Characters | Example |
| --- | --- | --- |
| Country Code | 2 Characters | ZA |
| Region Code | 1 Character | W |
| Location Code | 5 Characters | TECHC |
| Rack Type Code | 5 Characters | OMX60 Refer Table 55 Rack Type List |
| Instance Number | 3 digits | 004 |

Table 21 Rack Naming Components

#### Location Component

User will first select the site where the Rack is to be placed.

The Site will be created as per Table 3 Location Type Relationships

The Country Code, Province Code and the Location Code will be automatically derived from the site selected by the user while generating the Rack name using the Naming Convention Tool.

#### Rack Type Component

The list of Rack Type will be available as a drop down which the user will select using the Naming Convention tool. Refer Table 55 Rack Type List for Rack Type Codes. Type Code will be automatically generated by the tool while adding the new types as a part of metadata creation. The code provided in appendix is for example.

#### Instance Number

The Instance Number will be calculated automatically. The instance number will be incremented per rack under a particular location based on Rack Type.

### Rack Attributes

| Attribute Name | Attribute Type | Character Length | Example |
| --- | --- | --- | --- |
| Rack Position | Mandatory | 50 Characters | A:02 |
| Rack Owner | Mandatory | Enumeration | Vodacom Refer Table 65 Rack Owner |
| Rack Size | Mandatory | 50 Characters | 72U |

Table 22 Rack Attributes

#### Rack Position Attribute

The current Rack Naming Convention contains the rack position in the name. This component will be stored as an attribute to the rack. The attribute will be a free text box which the user will populate while generating the rack name using the Naming Convention Tool.

#### Rack Owner

This attribute will capture as to who owns the site i.e. if the site is owned by Vodacom, Telkom, MTN, Cell C, Telkom etc. The attribute will be a drop down which the user will fill while creating a site under a location using the naming convention tool. Refer Table 65 Rack Owner

#### Rack Size

This component will be stored as an attribute to the rack. The attribute will be a free text box which the user will populate while generating the rack name using the Tool.

## ODF / Patch Panel

### ODF / Patch Panel Naming Convention

Below is the proposed naming convention for ODF / Patch Panel.

Figure 29 ODF/ Patch Panel Naming Convention

### ODF / Patch Panel Naming Components

| Naming Components | No of Characters | Example |
| --- | --- | --- |
| Country Code | 2 Characters | ZA |
| Region Code | 1 Character | W |
| Location Code | 5 Characters | TECHC |
| ODF / Patch Panel Type Code | 5 Characters | ADCFL Refer Table 56 ODF Type List |
| Instance Number | 3 digits | 004 |

Table 23 ODF / Patch Panel Naming Components

#### Location Component

User will first select the site where the ODF / Patch Panel is to be placed.

The Site will be created as per Table 3 Location Type Relationships. The Country Code, Province Code and the Location Code will be automatically derived from the site selected by the user while generating the ODF / Patch Panel name using the Naming Convention Tool.

#### ODF / Patch Panel Type Component

The list of ODF / Patch Panel Type will be available as a drop down which the user will select using the Naming Convention tool. Refer Table 56 ODF / Patch Panel Type List for list of ODF / Patch Panel Types and Codes. Type Code will be automatically generated by the tool while adding the new types as a part of metadata creation. The code provided in appendix is for example.

#### Instance Number

The Instance Number will be calculated automatically. The instance number will be incremented per rack under a particular location based on ODF / Patch Panel Type.

### ODF / Patch Panel Attributes

| Attribute Name | Attribute Type | Character Length | Example |
| --- | --- | --- | --- |
| ODF / Patch Panel Position | Mandatory | 50 Characters | A:02 |
| Manhole Entry | Mandatory | 100 Characters | WGEOVM0001 |

Table 24 ODF / Patch Panel Attributes

#### ODF / Patch Panel Position Attribute

The current ODF / Patch Panel Naming Convention contains the ODF / Patch Panel position in the name. This component will be stored as an attribute to the ODF / Patch Panel. The attribute will be a free text box which the user will populate while generating the ODF / Patch Panel name using the Naming Convention Tool.

#### Manhole Entry Attribute

The attribute will be supported by a search box which will allow the user to find the backup link.

## ODF / Patch Panel Port

### ODF / Patch Panel Port Naming Convention

Below is the naming convention for ODF / Patch Panel Port currently implemented in Connect Master and will be used as-is.

Figure 30 ODF / Patch Panel  Port

### ODF / Patch Panel Port Naming Components

| Naming Components | No of Characters | Example |
| --- | --- | --- |
| Connection Point | 2 Characters | CP |
| Port Number | 3 Number | 001 |

Table 25 ODF / Patch Panel Port Naming Components

#### Connection Point Component

This will be generated by Connect Master automatically while creating an ODF/ ODR/ Patch Panel etc.

#### Port ID Component

This will be generated by Connect Master automatically while creating an ODF/ ODR/ Patch Panel etc.

### ODF / Patch Panel Port Attributes

| Attribute Name | Attribute Type | Character Length | Example |
| --- | --- | --- | --- |
| Bar Code | Optional | 10 Characters | AAA 440 |

Table 24 ODF / Patch Panel Port Attributes

#### Bar Code Attribute

This attribute will be captured by another project to detect the connectivity between two ports of ISP devices.

## Device Port (For ISP)

### Device Port Naming Convention

Below is the proposed naming convention for Device Port for ISP labelling. The naming will be as per current implementation of Connect Master.

Figure 32 Device Port Naming Convention

### Device Port Naming Components

| Naming Components | No of Characters | Example |
| --- | --- | --- |
| Port | 4 Characters | Port |
| Port Number | 3 Characters (Numeric only) | 001 |

Table 26 Device Port Naming Components

#### Port Component

Port will be a text prefix to ports of device.

#### Port Number Component

Port number will be auto allocated while creating an active device in Connect Master.

### Device Port Attributes

| Attribute Name | Attribute Type | Character Length | Example |
| --- | --- | --- | --- |
| Bar Code | Optional | 10 Characters | AAA 440 |

Table 24 ODF Port Attributes

#### Bar Code Attribute

This attribute will be captured by another project to detect the connectivity between two ports of ISP devices (ODF/ODR/Patch Panel/Active Equipment etc.).

## Patch Cord

### Patch Cord Naming Convention

Below is the proposed naming convention for Patch Cord.

Note: Kindly note that currently patch cord are not modelled in Connect Master and the below convention will be implemented once the modelling is available in Connect Master.

Figure 34 Patch Cord/Cable/Lead Naming Convention

### Patch Cord Naming Components

| Naming Components | No of Characters | Example |
| --- | --- | --- |
| A/Z End ODF Name | 16 Character | ZAWTECHCFAM14004 |
| A/Z End Device Name | 16 Characters | ZAWTECHCTL860005 |
| Instance Number | 3 Number | 002 |

Table 27 Patch Cord Naming Components

#### A/Z End ODF and Device Name Component

This will be the starting or terminating ODF/ODR/PP/Active equipment name for the patch cord/lead.

#### Instance Number

This will be the instance number of patch cord/lead within the originating and terminating devices.

### Patch Cord Attributes

| Attribute Name | Attribute Type | Character Length | Example |
| --- | --- | --- | --- |
| Link Number: WL | Optional (when available) | 10 Characters | Link Number: WL (WL00000034) for Wire Links as ordered within Transmission domain |

#### Link Number

The current Trunk Naming Convention captures the link number generated in TX Manager in the Trunk Name. This can also be captured as an attribute to the Patch Cord where it was allocated within the Transmission domain. The attribute will be a free text which the user will populate while generating the link name using the Naming Convention Tool.

## Manhole

### Manhole Naming Convention

Below is the proposed naming convention for Manhole for OSP labelling.

Figure 34 Manhole Naming Convention

### Manhole Naming Components

| Naming Components | No of Characters | Example |
| --- | --- | --- |
| Province Code | 1 Character | W |
| Manhole Owner | 2 Character | VM for Vodacom Manhole or Handhole |
| Instance Number | 5 Digits | 00042 |

Table 30 Manhole Naming Components

#### Province ID Component

Users will be provided with an option to select a Province ID to generate a new Manhole name. The manhole name will have Province as the first character.

#### Manhole Owner Component

Once user selects appropriate Country and province to generate a manhole name, they also need to select appropriate Manhole Owner. Manhole owner would be Vodacom, Telkom etc. Each manhole owner will be referred in the name with unique ID. The list of ID for each owner can be found in Table in appendix.

#### Instance Number Component

Instance number will be generated for each manhole sequentially and not based on the province selected. This instance number will be a 5 digit number and will be unique.

### Manhole Attributes

| Attribute Name | Attribute Type | Character Length | Example |
| --- | --- | --- | --- |
| Latitude | Mandatory | Number | -26.161997 |
| Longitude | Mandatory | Number | 28.210252 |
| Complete Address | Optional (if available) | 250 Characters | 12 Bussing Road, Aureus, Randfontein |
| Manhole Type | Mandatory | Drop Down | Draw box (600 x 600 x1000) Refer Table 66 Manhole Type List |
| Manhole Number | Mandatory | Free Text box |  |

Table 31 Rack Attributes

#### Latitude, Longitude and Complete Address Attribute

User needs to capture this attribute while generating manhole name. This attribute will enrich the manhole position information and will help field engineers to locate manhole position in case of any fault or any changes in network.

#### Manhole Types

User needs to capture this attribute while generating manhole name. This attribute will ensure the manhole type and thus will help in space reservations and planning. Refer Table 66 Manhole Type List

#### Manhole Number

User needs to capture this attribute while generating manhole name. This attribute will be useful to unlock the manhole.

## Pole for Overhead Fibre

### Pole Naming Convention

Below is the proposed naming convention for Poles for OSP labelling.

### Pole Naming Components

| Naming Components | No of Characters | Example |
| --- | --- | --- |
| Province Code | 1 Character | L for Limpopo Province |
| Manhole Owner | 2 Character | VP for Vodacom Pole |
| Instance Number | 5 Digits | 00002 |

Table 32 Manhole Naming Components

#### Province ID Component

Users will be provided with an option to select a Province ID to generate a new Pole name. The Pole name will have province as the first character.

#### Pole Owner Component

Once user selects appropriate Province to generate a Pole name, they also need to select appropriate Pole Owner. Pole owner would be Vodacom, Telkom etc. Each Pole owner will be referred in the name with unique ID. The list of ID for each owner can be found in the Table in appendix.

#### Instance Number Component

Instance number will be generated for each Pole sequentially and not based on the province selected. This instance number will be a 5 digit number and will be unique.

### Pole Attributes

| Attribute Name | Attribute Type | Character Length | Example |
| --- | --- | --- | --- |
| Latitude | Mandatory | Number | -26.161997 |
| Longitude | Mandatory | Number | 28.210252 |
| Complete Address | Optional (if available) | 250 Characters | 12 Bussing Road, Aureus, Randfontein |
| Pole Manufacturer | Mandatory | Drop Down | Rocla Aveng Refer Table 74 Pole Manufacturer and Specification List |
| Pole Specification | Mandatory | Drop Down linked to the Manufacturer field | 9m Hollow from Rocla 5,5kn 9m cast pole from Rocla 4kn Refer Table 74 Manhole Type List |

Table 29 Rack Attributes

#### Latitude, Longitude and Complete Address Attribute

User needs to capture this attribute while generating Pole name. This attribute will enrich the Pole position information and will help field engineers to locate Pole position in case of any fault or any changes in network.

#### Pole Types

User needs to capture this attribute while generating Pole name. This attribute will ensure the Pole type and thus will help in space reservations and planning. Refer Table 66 Pole Type List

#### Pole Specification

User needs to capture this attribute while generating Pole name. This attribute will be useful for inventory management. Refer Table 66 Pole Type List

## Splice Closure

### Splice Closure Naming Convention

There are two types of Splice Closure:

- Overlying: A splice Closure on Pole.

- Underlying: A splice Closure inside manhole.

Below is the proposed naming convention for splice closure for OSP labelling.

Figure 36a Manhole Splice Closure Naming Convention

Figure 36b Overlying Splice Closure on Pole Naming Convention

### Splice Closure Naming Components

| Naming Components | No of Characters | Example |
| --- | --- | --- |
| 1) Manhole Id (In case of Underlying) 2) Country & Province plus O (In case of Overlying) | 1) 10 Char in case of Manhole ID. 2) 4 Char Code. Country and Province Code plus an <O> for overlying Splice Closure Code. | ZAGVM0001 ZAGOxxxxxx |
| <S>Instance Number | 5 Character | S0001 |

Table 30 Splice Closure Naming Components

#### Country Code and Manhole ID or Province and Sub Place ID Component

Users will be provided with an option to select the Country and Province and type of Splice Closure to generate a new splice closure name. He will be provided with an option to select manhole name in case of splice closure located inside manhole. The splice closure name will have the country and province as the first 3 characters with manhole number in case of splice closure inside manhole or ‘x’ as a filler to make the component of 10 characters

#### Instance Number Component

Instance number will be generated for each splice closure based on the province selected. This instance number will be a 4 digit number and will be unique within the province selected.’ S’ will be appended to instance number to make to a splice closure instance number.

### Splice Closure Attributes

| Attribute Name | Attribute Type | Character Length | Example |
| --- | --- | --- | --- |
| Latitude | Mandatory | Number | -26.161997 |
| Longitude | Mandatory | Number | 28.210252 |
| Complete Address | Mandatory | 250 Characters | 12 Bussing Road, Aureus, Randfontein |
| Pole Name | Mandatory | 50 Characters | LVP00002 |

Table 31 Splice Closure Attributes

#### Latitude, Longitude and Complete Address Attribute

User needs to capture this attribute while generating the Pole name. This attribute will enrich the Pole position information and will help field engineers to locate the Pole position in case of any fault or any changes in network.

#### Pole Name Attribute

User needs to select this attribute from the existing pole names, while generating splice closure name. This attribute will enrich the splice closure position information and will help field engineers to locate splice closure position in case of any fault or any changes in network.

## Duct

### Duct Naming Convention

Below is the proposed naming convention for the duct for OSP labelling.  Duct labelling will only be required for Vodacom owned ducts.  Leased fibre will not carry this information.

Figure 35 Duct Naming Convention

### Duct Naming Components

| Naming Components | No of Characters | Example |
| --- | --- | --- |
| Network Type | 1 Character | N or C etc Network Type Code List |
| A-End and Z-End Locations Codes | 8 Characters each | ZAGTECHP |
| Duct Type Abbreviation | 6 Characters | Duct Type |
| Duct Counter | 1 Character | 1, 2, etc |

Table 32 Cable Naming Components

#### Network Type Component

This component explains whether the cable is a National Cable or a Core Cable. Users’ needs to select one of the values from a list of values to provide the network type for Cable.

#### A-End and Z-End Component

Users will be provided to select A-End Site, A-End Type (like ODF, Manhole and Splice Closure) and then A-End Point. The same needs to be done for Z-End.  These A-End and Z-End will be part of Cable name.

#### Duct Type Component

Users will be provided with a list of duct types to select appropriate duct for the trench for which name needs to be generated. For existing Duct Types, the codes will be loaded as prescribed in the document. Type Code will be automatically generated by the tool while adding the new types as a part of metadata creation. List of Duct types which will be provided to users are listed in appendix section. Duct Type.

#### Duct Counter

Duct between the same A-end and Z-End locations will be numbered as 1, 2 and so on.  This number will be generated for each additional Duct of the same type based on the A-End and Z-End selected. This Duct  number will be a 1 Character and will be unique within the same two terminating points.

### Duct Attributes

| Attribute Name | Attribute Type | Character Length | Example |
| --- | --- | --- | --- |
| Civil Works Contractor | Mandatory | Enumerations | MCT Civil works/Fibre Build Contractor List |
| Route Request Type | Mandatory | Enumerations | Self-Build (SB) Refer Table 63 Cable Request Type |

Table 33 Splice Closure Attributes

#### Civil Works Contractor Attribute

This attribute is related to the Vendor which were involved in the civil work. It is important to label the duct in an intuitive manner in case there are faults reported.  User needs to capture this attribute while generating Duct Name. This is going to be a one of the list of values and will be mandatory attribute to store information. Refer Civil works/Fibre Build Contractor List

#### Route Request Type Attribute

This attribute is related to the information regarding the ownership of the route in case of faults. Values like SB for Self Build, CB for Co-Build or VCW for Civil Builds will be available for users to select one of them. Refer Table 63 Cable Request Type

## Trench

### Trench Route Naming Convention

Below is the proposed naming convention for the trench route for ConnectMaster names and OSP labelling.  Trench labelling will only be required for fibre routes (Vodacom owned and co-build trenches).  Trench route will be group of Trench Sectors showing the actual route. Trenches are named in trench sectors from manhole to manhole.

Figure 36 Trench Route Naming

### Trench Route Naming Components

| Naming Components | No of Characters | Example |
| --- | --- | --- |
| Network Type | 1 Character | N or C etc (Table) |
| Route A-End Location Codes | 8 Characters | ZAGPPQPK |
| Route Z-End Location Codes | 8 Characters | ZAGPSIVT |
| Trench Number (Counter) | 2 Characters | T1, T2,…… |

Table 34 Trench Naming Components

#### Network Type Component

This component explains whether the cable is a National Cable or a Core Cable. Users’ needs to select one of the values from a list of values to provide the network type for Cable.

#### A-End and Z-End Locations of Route Component

Users will be provided to select A-End Location and a Z-end location to identify the parent route.  Then they have to select the location where the trench started to the first manhole, this must continue from manhole to manhole until the Z-end location of the route are selected.

#### Trench Number (Counter) Component

Trenches between the same A-end and Z-End locations will be numbered as R01, R02 and so on.  This number will be generated for each additional trench based on the A-End and Z-End selected. This Trench number will be a 1 Character + 2 digit number and will be unique within the same two terminating points.

### Trench Attributes

| Attribute Name | Attribute Type | Character Length | Example |
| --- | --- | --- | --- |
| Location / Manhole Latitude | Automated when selected | Number | -26.161997 |
| Location / Manhole Longitude | Automated when selected | Number | 28.210252 |
| Civil Works Contractor | Mandatory | Enumerations | MCT Civil works/Fibre Build Contractor List |
| Route Request Type | Mandatory | Enumerations | Self-Build (SB) Refer Table 63 Cable Request Type |

Table 35 Splice Closure Attributes

#### Latitude and Longitude Attribute

User needs to capture this attribute while generating the Pole name. This attribute will enrich the Pole position information and will help field engineers to locate the Pole position in case of any fault or any changes in network.

#### Civil Works Contractor Attribute

This attribute is related to the Vendor which were involved in the civil work. It is important to label the trench in an intuitive manner in case there are faults reported.  User needs to capture this attribute while generating Trench Name. This is going to be a one of the list of values and will be mandatory attribute to store information. Refer Civil works/Fibre Build Contractor List Error! Reference source not found.

#### Route Request Type Attribute

This attribute is related to the information regarding the ownership of the route in case of faults. Values like SB for Self Build, CB for Co-Build or VCW for Civil Builds will be available for users to select one of them. Refer Table 63 Cable Request Type

### Trench Sector Naming Convention

Below is the proposed naming convention for the trench for ConnectMaster names and OSP labelling.  Trench labelling will only be required for fibre routes (Vodacom owned, co-build and leased trenches).  Trenches are named in trench sectors from manhole to manhole.

Trench Sector will be child of Trench route in Connect Master

Figure 37 Trench Sector Naming Convention

### Trench Sector Naming Components

| Naming Components | No of Characters | Example |
| --- | --- | --- |
| Start Manhole/Location Codes | 8 Characters | ZAGPPQPK |
| End Manhole/Location Codes | 8 Characters | ZAGPSIVT |

Table 34 Trench Naming Components

#### Trench Start Component (Manhole/Location)

Users will have to select the trench start Location and the first manhole, this is seen as the trench child route which belongs to the parent route.  The child sections of the route are captured from manhole to manhole until the Z-end location has been reached.  This naming capability currently exists in TxMan and is created during the capturing of routes on the map (GEO interface).

#### Trench End Component (Manhole/Location)

Users will have to select the trench start Location and the first manhole, this is seen as the trench child route which belongs to the parent route.  The child sections of the route are captured from manhole to manhole until the Z-end location has been reached.  This naming capability currently exists in TxMan and is created during the capturing of routes on the map (GEO interface).

## Cable

### Cable Naming Convention

Below is the proposed naming convention for cable for OSP labelling.

Figure 38 Cable Naming Convention

### Cable Naming Components

| Naming Components | No of Characters | Example |
| --- | --- | --- |
| Network Type | 1 Character | N or C etcNetwork Type Code List |
| A-End and Z-End Locations | 16 Characters each | ZAGTECHPXXXXXXXX |
| Cable Owner | 3 Characters | VOD / DFA etc |
| Cable Type | 5 Characters | G652D Refer Table 57 Cable Type List |
| Cable (Fibre) Size | 4 Characters | 024F Refer Table 60 Fibre Size List |
| VOF, WL or CL | 10 Characters | VOF0000322 |

Table 36 Cable Naming Components

#### Network Type Component

This component explains whether the cable is a National Cable or a Core Cable. Users’ needs to select one of the values from a list of values to provide the network type for Cable.

#### A-End and Z-End Component

Users will be provided to select A-End Site, A-End Type (which can be one of the following:  ODF, Manhole, Splice Closure and Nodes) and then A-End Point. The same needs to be done for Z-End.  These A-End and Z-End will be part of Cable name.

#### Cable Owner Component

Users will be provided with a list of cable owners to select appropriate cable owner for the cable for which name needs to be generated. List of cable owners which will be provided to users are listed in appendix section.  Cable Owner List

#### Cable Type Component

Users will be provided with a list of cable types to select appropriate cable type for the cable for which name needs to be generated. For existing Cables Types, the codes will be loaded as prescribed in the document. Type Code will be automatically generated by the tool while adding the new types as a part of metadata creation. List of cable type which will be provided to users are listed in appendix section. Refer Table 57 Cable Type List

#### Fibre Size Component

Users will be provided with a list of Fibre Sizes to select appropriate Fibre Size for the cable for which name needs to be generated. List of Fibre Size which will be provided to users are listed in appendix section. Refer Table 60 Fibre Size List

#### Instance Number Component

Instance number will be generated for each Cable based on the A-End and Z-End selected. This instance number will be a 3 digit number and will be unique within the same two terminating points.

#### Cable (Link) Number Component

This cable number is related to the Link Number (VOF Number) which gets generated in TX Manager for order purpose. This is going to be help in identifying the relationship between the various impacted circuit and the fibre cut using this Link Number. Note that we can harvest the same information using the relationship of Link to Circuit in various other tools.

### Cable Attributes

| Attribute Name | Attribute Type | Character Length | Example |
| --- | --- | --- | --- |
| Cable Provider | Mandatory | Enumerations | Dark Fibre Africa (DFA) Civil works/Fibre Build Contractor List Error! Reference source not found. |
| Cable Request Type | Mandatory | Enumerations | Self-Build (SB) Refer Table 63 Cable Request Type |
| Cable Position | Mandatory | Enumerations | Overlying (O) Underlying (L) |
| Cable Manufacturer | Mandatory | Enumerations | Samsung Refer Table 58 Cable Supplier List |
| Business Area / Internal Customer | Mandatory | 3 Characters | Enterprise Business Unit Refer Civil works/Fibre Build Contractor List |

Table 37 Splice Closure Attributes

#### Cable Provider Attribute

This attribute is related to the Vendor which provided the cable. In case of any fault in that cable, the cable provided should be contacted. User needs to capture this attribute while generating Cable Name. This is going to be a one of the list of values and will be mandatory attribute to store information. Refer Civil works/Fibre Build Contractor List

#### Cable Request Type Attribute

This attribute is related to the information regarding the ownership of cable in case of faults. Values like SB for Self Build, CB for Co-Build will be available for users to select one of them. Refer Table 63 Cable Request Type

#### Cable Position Attribute

This attributes stores the information like cable is an Overlying or Underlying cable. This will help in localising the fault. Only two values will be part of this attribute as Overlying and Underlying.

#### Cable Manufacturer Attribute

This attribute stores the information like cable is manufactured by Samsung or any other company. Refer Table 58 Cable Supplier List

#### Business Area / Internal Customer

The Cable naming convention need to include the Business Area / Internal Customers.

Business Area / Internal Customer will appear as a drop down to users when they are generating the cable name using the Naming Convention Tool. Refer Business Area / Internal Customer

# References

## Naming Convention Tool

The Naming Convention Tool will be used as a centralised tool by all the Network Business User Groups for generation of network object names. The names will be generated as per the convention given in Section 2 Design .

A Naming Convention Tool that was intended to create and maintain the names as defined within this document has been developed. This tool is deemed cumbersome since it:

- Requires that an additional tool is used

- Requires data to be copied into it

- Requires data to be copied back into existing tools used in process

It has been decided that the Naming Convention be implemented in ConnectMaster for the time being. The naming convention & algorithms defined in this document will be implemented.

# Appendix

## Country Code

| Country | Country Code |
| --- | --- |
| South Africa | ZA |
| Tanzania | TZ |
| Lesotho | LS |
| DRC | CG |
| Mozambique | MZ |

Table 43 Country Code		_Country_Attributes

## ZA Province Code

| Province Name | Province Code |
| --- | --- |
| Gauteng | G |
| North West | N |
| Northern Cape | C |
| Free State | F |
| Western Cape | W |
| Mpumalanga | M |
| Limpopo | L |
| Eastern Cape | E |
| Kwazulu Natal | K |

Table 44 Province Code		_Province_Attributes

## LS Province (District) Code

| Province Name | Province Code |
| --- | --- |
| Berea | B |
| Butha-Buthe | U |
| Leribe | L |
| Mafeteng | M |
| Maseru | A |
| Mohale’s Hoek | O |
| Mokhotlong | K |
| Qacha’s Nek | Q |
| Quthing | H |
| Thaba-Tseka | T |

Table 44 Province Code		_ LS Province (District)_Attributes

## Power Grid Area

Province Name

Caledon Area

Mankweng Area

Hazyview Area

Mossel Bay n Plett Area

Nelspruit Area

Humansdorp Area

George Area

Randburg Area

Midrand Area

Tlokweng Area

Soshanguve Area

Porterville Area

Ga_Rankuwa Area

Glengarry Area

Phokwane Area

Mafikeng Area

Warmbaths Area

Chatsworth Area

Irene Area

Mokopane Area

Citrusdal Area

Diswilmar Area

Eshowe Area

Hlabisa Area

Brooklyn Area

Bethlehem Area

Lynnwood Area

Karoo Area

Thohoyandou Area

Table 44 Power Grid Areas

## Location Code Generation Algorithm

The following algorithm will be used to generate the Location Codebased on a Location Name provided. The algorithm will ensure that the Location Code generated remains unique within each Province.

- <First Character > <Second Character> <Third Character> <Fourth Character> <Fifth Character> - “ ABCDE”

- <First Character > <Second Character> <Third Character> <Fourth Character> <Sixth Character> - “ ABCDF”

- <First Character > <Second Character> <Third Character> <Fourth Character> <Seventh Character> - “ ABCDG”

- <First Character > <Second Character> <Third Character> <Fifth Character> <Sixth Character> - “ ABCEF”

- <First Character > <Second Character> <Third Character> < Sixth Character> <Seventh Character> - “ ABCFG”

- <First Character > <Second Character> <Third Character> < Seventh Character> <Eighth Character> - “ABCGH”

- <First Character > <Third Character> <Second Character> <Fourth Character> <Fifth Character>  - “ ACBDE”

- <First Character > <Third Character> <Fourth Character> <Fifth Character> <Sixth Character>  - “ ACDEF”

- <First Character > <Third Character> <Fourth Character> <Fifth Character> <Second Character>  - “ ACDEB”

- <First Character > <Fourth Character> <Second Character> <Third  Character> <Fifth Character>  - “ ADBCE”

- <First Character > <Fourth Character> <Second Character> <Third  Character> <Sixth  Character>  - “ ADBCF”

- <First Character > <Fourth Character> <Third Character> <Second Character> <Sixth  Character>  - “ ADCBF”

- <First Character > <Fourth Character> <Third Character> <Second Character>  <Fifth Character>   - “ ADCBE”

- <First Character > <Fifth Character> <Fourth Character> <Third Character> <Second Character>  - “ AEDCB”

- <First Character > <Fifth Character> <Fourth  Character> <Second Character>  <Third Character>   - “ AEDBC”

- <First Character >  <Sixth  Character> <Fifth Character> <Fourth Character> <Third Character>  - “ AFEDC”

- <First Character > <Sixth  Character> <Fifth Character> <Second Character>   <Third Character>  - “ AFEBC”

- <First Character > <Seventh  Character> <Sixth Character> <Fifth Character>   <Fourth Character>  - “ AGFED”

- <First Character > <Seventh  Character> <Sixth Character> <Fifth Character>   <Third Character>  - “ AGFEC”

- <First Character > <Seventh  Character> <Sixth Character> <Fifth Character>   <Second Character>   - “ AGFEB”

- <First Character > <Second Character> <Third Character> <Fourth Character> < Numeric Number (1 to 9)>  – “ABCD1”

- <First Character > <Third Character> <Fourth Character> <Fifth Character> < Numeric Number (1 to 9)>  - “ ACDE1”

- <First Character > <Second Character> <Third Character> <Fifth Character> < Numeric Number (1 to 9)>  - “ ABCE1”

- <First Character > <Second Character> <Third Character> <Two digit Numeric Number ( 01 to 99 )>  - “ ABC01”

- <First Character > <Second Character> <Fourth Character>  <Two digit Numeric Number ( 01 to 99 )>  -  “ ABD01”

- <First Character > <Third Character> <Fourth Character> <Two digit Numeric Number ( 01 to 99 )>  -  “ ACD01”

- For location name with TWO words:

- The first three characters from the first word will be concatenated with the second word,

- Then the first 5 characters of this result will then be sent through the “attempts” as specified above.

Example

Techno Centre

- TEC + CENTRE = TECCENTRE

- TECCE sent through algorithm

- For location name with MORE than TWO words:

- The first two characters from the first word will be concatenated with the second word,

- Then the first 3 characters of this result will be concatenated with the third word,

- Then the first 5 characters of this result will then be sent through the “attempts” as specified above.

Example

Techno Mamparra Centre

a)	TE + MAMPARRA = TEMAMPARRA

b) 	TEM + CENTRE = TE

c)  	TEMCE sent through algorithm

## Type of site

Type of Site

Core

Edge

Aggregation

Access

Hubstation

Table 45 Type of Site

## Site Owner

Site Owner

Vodacom

Telkom

MTN

Cell C

Neotel

Fiberco

Teraco

Internet Solutions

Table 46 Site Owner

## Vodacom Operating Region

| Operating Region | Abbreviation |
| --- | --- |
| Central | CEN |
| Eastern | EAS |
| Kwazulu-Natal | KZN |
| Limpopo | LIM |
| Mpumalanga | MPU |
| Northern Gauteng | NGA |
| Southern Gauteng Central | SGC |
| Southern Gauteng South | SGS |
| Western | WES |

Table 47 Operating Region

## POC Type

POC Type

POC 1

POC 2

POC 3

POC 4 (Tail)

Table 43 POC Type for Site

## Node / Device Type Code

| Vendor | Vendor Mode | Code |
| --- | --- | --- |
| Alcatel-Lucent | ULS | ALxULS |
| Alcatel-Lucent | DWDM 1830PSS-36 | ALPS36 |
| Alcatel-Lucent | DWDM 1830PSS-32 | ALPS32 |
| Alcatel-Lucent | DWDM 1830PSS-64 | ALPS64 |
| Alteon | Alteon Application Switch 2208 | AL2208 |
| Alteon | Alteon Application Switch 2424-SSL | AL2424 |
| Bay Wellfleet | s72033 | BW7203 |
| CISCO | 3702P | CS3702 |
| CISCO | 3600 | CS3600 |
| CISCO | 2700 | CS2700 |
| CISCO | 1530 | CS1530 |
| CISCO | 702W | CS702W |
| CISCO | 3700I | CS370I |
| Cisco | 870 | CSX870 |
| Cisco | 1530 | CS1530 |
| Cisco | 1841 | CS1841 |
| Cisco | 2700 | CS2700 |
| Cisco | 2811 | CS2811 |
| Cisco | 2900 | CS2900 |
| Cisco | 2960 | CS2960 |
| Cisco | 3550 | CS3550 |
| Cisco | 3560 | CS3560 |
| Cisco | 3600 | CS3600 |
| Cisco | 3700 | CS3700 |
| Cisco | 3725 | CS3725 |
| Cisco | 3750 | CS3750 |
| Cisco | 3825 | CS3825 |
| Cisco | 3845 | CS3845 |
| Cisco | 3900 | CS3900 |
| Cisco | 4506 | CS4506 |
| Cisco | 4510 | CS4510 |
| Cisco | 4948 | CS4948 |
| Cisco | 6509 | CS6509 |
| Cisco | 7200 | CS7200 |
| Cisco | 7206 | CS7206 |
| Cisco | 7300 | CS7300 |
| Cisco | 7301 | CS7301 |
| Cisco | 7304 | CS7304 |
| Cisco | 7609 | CS7609 |
| Cisco | 7613 | CS7613 |
| Cisco | 12404 | C12404 |
| Cisco | 12406 | C12406 |
| Cisco | 1600 Software | CS1600 |
| Cisco | 2404/PRP | CS2404 |
| Cisco | 2406/PRP | CS2406 |
| Cisco | 2800NM | CS2800 |
| Cisco | 3560E | C3560E |
| Cisco | 3602P | CS3602 |
| Cisco | 3700E | CS370E |
| Cisco | 3700I | CS370I |
| Cisco | 3702P | CS3702 |
| Cisco | 3750E | C3750E |
| Cisco | 3900e | C3900e |
| Cisco | 4900M | C4900M |
| Cisco | 702W | CS702W |
| Cisco | 7200P | C7200P |
| Cisco | 7600rs | CS76rs |
| Cisco | 7600s | CS760s |
| Cisco | ASR 1006 | CA1006 |
| Cisco | ASR 5000 | CA5000 |
| Cisco | ASR 9000 | CA9000 |
| Cisco | ASR 9001 | CA9001 |
| Cisco | ASR 9006 | CA9006 |
| Cisco | ASR 9010 | CA9010 |
| Cisco | BS30X0 | CS30X0 |
| Cisco | C2950 | CC2950 |
| Cisco | CRS 3 | CSCRS3 |
| Cisco | Firewall | CSFRWL |
| Cisco | GS Software | CSGSSW |
| Cisco | m9500 | Cm9500 |
| Cisco | n7000 | CN7000 |
| Cisco | PPC_LINUX_IOSD | CPPCLX |
| Cisco | s2t54 | Cs2t54 |
| Cisco | s3223 | Cs3223 |
| Cisco | SAMI | CSSAMI |
| Cisco | vg224 | Cvg224 |
| Cisco | WS | CSXXWS |
| Cisco | X86_64_LINUX_IOSD | CX8664 |
| Coriant | 7090-05 CE | CE9005 |
| Coriant | 7090-07 CE | CE9007 |
| Coriant | 7090-15 CEM | CE9015 |
| Coriant | 7090-20 CEM | CE9020 |
| Coriant | 7090-25 CE | CE9025 |
| Coriant | 7090-40 CEM | CE9040 |
| Coriant | 7090-45 CE | CE9045 |
| Coriant | 7090-60 CEM | CE9060 |
| Ericsson | AMM 20P | ERA20P |
| Ericsson | AMM 20PB | ERA20B |
| Ericsson | AMM 2PB | ERA2PB |
| Ericsson | AMM 6PC | ERA6PC |
| Ericsson | AMM 6PD | ERA6PD |
| Huawei | BSC 6900 GSM | HWB69G |
| Huawei | BSC 6900 UMTS | HWB69U |
| Huawei | BTS 3900 | HWB390 |
| Huawei | BTS 3900A | HWB39A |
| Huawei | BTS 3900E | HWB39E |
| Huawei | DBS 3900 | HWD390 |
| Hughes | HT 9800 | HT9800 |
| Hughes | RT 9800 | RT9800 |
| Ironport |  | IRNPRT |
| Juniper | Juniper Firewall | JNPRFW |
| Juniper | SRX5800 | JSRX58 |
| Nera | Evolution | NEEVOT |
| Nera | Evolution Access | NEEVAC |
| Nera | Evolution Edge | NEEVED |
| Nera | Evolution XPAND IP | NEEVXP |
| Nexus | Nexus 5548 | NX5548 |
| Nexus | Nexus 2232 | NX2232 |
| Nexus | Nexus 2248 | NX2248 |
| Nexus | Nexus 6001 | NX6001 |
| Nortel | Layer2-7 GbE Switch Module | NLGbES |
| Nortel | Nortel Application Switch 2208 | NL2208 |
| Nortel | Nortel Application Switch 2424 | NL2424 |
| Nortel | Nortel Application Switch 3408 | NL3408 |
| NSN | ATCA (PLANNED) | NSATCA |
| NSN | BSC3I | NSBS3I |
| NSN | CSDB (PLANNED) | NSCSDB |
| NSN | D1800 | NSND18 |
| NSN | D900 | NSND9K |
| NSN | DX200 | NSDX20 |
| NSN | FLEXI MULTIRADIO | NSFXMR |
| NSN | HIS700 | NSNHIS |
| NSN | IPA2800 | NSIP28 |
| NSN | IPA800 | NSA800 |
| NSN | NB 341 | NSB341 |
| NSN | NB 381 | NSB381 |
| NSN | NB 440 | NSNB44 |
| NSN | NB 880 | NSNB88 |
| SIAE | ALC Plus2 | SIALP2 |
| SIAE | ALC Plus2e | SIAP2E |
| Tellabs | 6310 | TL6310 |
| Tellabs | 6325 | TL6325 |
| Tellabs | 6340 | TL6340 |
| Tellabs | 6345 | TL6345 |
| Tellabs | 6350 | TL6350 |
| Tellabs | 7100 | TL7100 |
| Tellabs | 7305 | TL7305 |
| Tellabs | 7325 | TL7325 |
| Tellabs | 7345 | TL7345 |
| Tellabs | 8120 | TL8120 |
| Tellabs | 8130 | TL8130 |
| Tellabs | 8150 | TL8150 |
| Tellabs | 8160 | TL8160 |
| Tellabs | 8170 | TL8170 |
| Tellabs | 8188 | TL8188 |
| Tellabs | 8602 | TL8602 |
| Tellabs | 8605 | TL8605 |
| Tellabs | 8606 | TL8606 |
| Tellabs | 8609 | TL8609 |
| Tellabs | 8615 | TL8615 |
| Tellabs | 8620 | TL8620 |
| Tellabs | 8630 | TL8630 |
| Tellabs | 8660 | TL8660 |
| Tellabs | 8665 | TL8665 |
| Tellabs | 8830 | TL8830 |
| Tellabs | 8840 | TL8840 |
| Tellabs | 7100 nano | TL71NA |
| Tellabs | 8860-R2 | TL88R2 |
| Coriant | 6310 | CR6310 |
| Coriant | 6325 | CR6325 |
| Coriant | 6335 | CR6335 |
| Coriant | 6340 | CR6340 |
| Coriant | 6350 | CR6350 |
| Coriant | 6350 | CR6350 |
| Coriant | 7100 NANO | CR71NA |
| Coriant | 7100 OTS | CR71TS |
| Coriant | CR7090-05CE | CR05CE |
| Coriant | CR7090-34G | CR034G |
| Coriant | CR7090-25CE | CR25CE |
| Coriant | CR7090-146G | CR146G |
| Coriant | CR7090-45CE | CR45CE |
| Coriant | 8120 | CR8120 |
| Coriant | 8130 | CR8130 |
| Coriant | 8150 | CR8150 |
| Coriant | 8160 | CR8160 |
| Coriant | 8170 | CR8170 |
| Coriant | 8188 | CR8188 |
| Coriant | 8602 | CR8602 |
| Coriant | 8605 | CR8605 |
| Coriant | 8606 | CR8606 |
| Coriant | 8609 | CR8609 |
| Coriant | 8611 | CR8611 |
| Coriant | 8615 | CR8615 |
| Coriant | 8620 | CR8620 |
| Coriant | 8625 | CR8625 |
| Coriant | 8630 | CR8630 |
| Coriant | 8635 | CR8635 |
| Coriant | 8660 | CR8660 |
| Coriant | 8665 | CR8665 |
| Coriant | 8830 | CR8830 |
| Coriant | 8840 | CR8840 |
| Coriant | 8860-R2 | CR88R2 |
| TippingPoint IPS |  | TIPIPS |
| XpressPass |  | XPRPAS |

Table 49 Device Type Code

## Business Area / Internal Customer

Business Area / Internal Customer

Vodacom Mobile (Network)

Enterprise Business Unit

Vodacom Retail (Vodashop)

Vodacom Wholesale

Vodacom Call Centre

Table 50 Business Area / Internal Customer

## Node Status

| Status | Description |
| --- | --- |
| Planned | When the Node is planned by the Planning team |
| Tested | When the Node is ATP’ed |
| In Service | When the Node is live in the Network |
| Faulty | When the Node is not functioning correctly and is faulty |
| Available | When the Node is placed in the Warehouse |
| To Be Built | When the node is provisioned |
| Decommissioned | When the node is removed from the network |

Table 46 Node Status

## Node Functionality

Node Functionality

BTS

BSC

RNC

NodeB

eNodeB

WiFi

MME

STP

GGSN

SGSN

MSC

HLR

MGW

CDS

WS

Router

Switch

Layer 3

Layer 4

AP

Firewall

P (Provider Router)

PE (Provider Edge Router)

PPE (Shared Provider and Provider Edge Router)

SLA (Customer facing SLA Router)

MGR (Core or Core/Customer facing SLA Router)

Table 53 Node Functionality

## Link Medium

| Link Medium | Medium Code |
| --- | --- |
| Fibre | FIB |
| Copper | COP |
| Microwave | M/W |
| Radio | RAD |
| Co-Axial | COX |

Table 54 Link Type and Link Type Code

## Link Bandwidth and Bandwidth Code

| Bandwidth Value | Bandwidth Code |
| --- | --- |
| 1/0 2M | 2Mxxx |
| 1/0 8M | 8Mxxx |
| 10M | 10Mxx |
| 100M | 100Mx |
| 1G | 1Gxxx |
| 10G | 10Gxx |
| 4M | 4Mxxx |
| 5M | 5Mxxx |
| 6M | 6Mxxx |
| 7M | 7Mxxx |
| 8M | 8Mxxx |
| 9M | 9Mxxx |
| 10M | 10Mxx |
| 40M | 40Mxx |
| 170M | 170Mx |
| 340M | 340Mx |
| STM-1 | ST1xx |
| STM-16 | ST16x |
| STM-4 | ST4xx |
| STM-64 | ST64x |
| OTM44 | OTM44 |
| OTM88 | OTM88 |

Table 55 Link Bandwidth and Bandwidth Code

## Protection Type

Protection Type

Primary

Secondary

Table 56 Protection Type

## Link Operator Type

Operator Type

Vodacom

Telkom

DFA

MTN

Neotel

Cell C

Teraco

WACS

Seacom

Eassy

Table 57 Link Operator Type

## Link Status

Link Status

Planned

Installed

Tested

In Service

Faulty

Decommissioned

Table 58 Link Status

## Circuit Function Code

Note: Where ever there is a “YES” in multiplier required, bandwidth will be based on the multiplier selected while creating the circuit. “XXXX” should be replaced with bandwidth values calculated.

| Logical Link Type | Bandwidth | Multiplier Required | Circuit Function Code |
| --- | --- | --- | --- |
| 1/0 VC-12 Trunk | VC12 | NO | VC12xxxx |
| 1/0 VC-12-mc Trunk | VC12 | YES | V12CXXXX |
| 1/0 VC-2-mc Trunk | VC2 | YES | V2CxXXXX |
| 4/1 VC4 Trunk | VC4 | NO | VC4xxxxx |
|  | VC4C4 |  | VC4C4xxx |
|  | VC4C16 |  | VC4C16xx |
|  | VC4C64 |  | VC4C64xx |
|  | VC4C256 |  | VC4C256x |
| 4/1 VC4 Trunk(Ethernet) | VC4 | YES | V4CxXXXX |
| 4/1 VC-4 External Trunk | VC4 | NO | VC4xxxxx |
| 1/0 8M Trunk | 8M | NO | PDH8Mxxx |
| OCH Trunk | 10G | NO | OCH10Gxx |
| OCH Trunk | 40G | NO | OCH40Gxx |
| OCH Trunk | 100G | NO | OCH100Gx |
| SDH tu12 Circuit | VC12 | NO | TU12xxxx |
| SDH tu12 Circuit (Ethernet) | VC12 | YES | T12CXXXX |
| SDH tu3 Circuit | VC3 | NO | TU3xxxxx |
| SDH tu3 Circuit (Ethernet) | VC3 | YES | TU3CXXXX |
| SDH tu2 Circuit | VC2 | NO | TU2xxxxx |
| SDH tu2 Circuit (Ethernet) | VC2 | YES | TU2CXXXX |
| SDH AU4 Circuit | VC4 | NO | AU4xxxxx |
| SDH AU4 Circuit (Ethernet) | STM-4 | YES | AU4CXXXX |
| PDH 1/0 Circuit | 64K | NO | PDH64Kxx |
|  | 128K |  | PDH128Kx |
|  | 192K |  | PDH192Kx |
|  | 256K |  | PDH256Kx |
|  | 320K |  | PDH320Kx |
|  | 384K |  | PDH384Kx |
|  | 448K |  | PDH448Kx |
|  | 512K |  | PDH512Kx |
|  | 576K |  | PDH576Kx |
|  | 640K |  | PDH640Kx |
|  | 704K |  | PDH704Kx |
|  | 768K |  | PDH768Kx |
|  | 832K |  | PDH832Kx |
|  | 896K |  | PDH896Kx |
|  | 960K |  | PDH960Kx |
|  | 1024K |  | PDH1024K |
|  | 1088K |  | PDH1088K |
|  | 1152K |  | PDH1152K |
|  | 1216K |  | PDH1216K |
|  | 1280K |  | PDH1280K |
|  | 1344K |  | PDH1344K |
|  | 1408K |  | PDH1408K |
|  | 1472K |  | PDH1472K |
|  | 1536K |  | PDH1536K |
|  | 1600K |  | PDH1600K |
|  | 1664K |  | PDH1664K |
|  | 1728K |  | PDH1728K |
|  | 1792K |  | PDH1792K |
|  | 1856K |  | PDH1856K |
|  | 1920K |  | PDH1920K |
|  | 1984K |  | PDH1984K |
|  | 2M |  | PDH2Mxxx |
| Frame Relay Virtual Circuit | 64K |  | FR64Kxxx |
|  | 640K |  | FR640Kxx |
|  | 960K |  | FR960Kxx |
|  | 65M |  | FR65Mxxx |
| Frame Relay Trunk | 128K | NO | FRT128Kx |
|  | 192K |  | FRT192Kx |
|  | 256K |  | FRT256Kx |
|  | 384K |  | FRT384Kx |
|  | 512K |  | FRT512Kx |
|  | 960K |  | FRT960Kx |
|  | 1M |  | FRT1Mxxx |
|  | 1.5M |  | FRT1.5Mxx |
| PPP Trunk | NA | NO | PPPxxxxx |
| VLAN Trunk | NA | NO | VLANxxxx |
| VLAN VPN | NA | NO | VLVPNxxx |
| ATM_L2VPN | TBF(Pierre) | NO | ATML2 |
| Ethernet L2 VPN | TBF(Pierre) | NO | ETHL2 |
| TDM L2 VPN | TBF(Pierre) | NO | TDML2 |
| BGPMPLSVPN |  | NO | IPVPN |
| S-VLAN | NA | NO | SVLANxxx |
| Pseudowire | NA | NO | PWxxxxxx |
| IP/MPLS Trunk | NA | NO | MPLSxxxx |
| Tunnel | NA | NO | MTUNxxxx |
| O & M | NA | NO | ONMxxxx |
| Traffic | NA | NO | TRFxxxxx |
| Signalling | NA | NO | SIGNxxxx |
| ABIS | NA | NO | ABISxxxx |
| IUB | NA | NO | IUBxxxxx |
| ATER | NA | NO | ATERxxxx |
| A | NA | NO | Axxxxxxx |
| IUPS | NA | NO | IUPSxxxx |
| IUCS | NA | NO | IUCSxxxx |
| IUR | NA | NO | IURxxxxx |
| MTP3 Link Set | NA | NO | MTP3LSxx |
| M3UA Link Set | NA | NO | M3UALSxx |
| STP Link | NA | NO | STPxxxxx |
| SS7 | NA | NO | SS7xxxxx |
| Circuit Group | NA | NO | CKTGRPxx |
| Trunk Group | NA | NO | TRKGRPxx |

Table 59 Circuit Function Code

## Bandwidth to Value Mapping

Based on the user selected multiplier supplied, the actual bandwidth will be calculated as indicated in the value column.

Example:  If user selects the circuit type as SDH AU4 Circuit (Ethernet), thus Bandwidth is STM-4 and multiplier is 4.  Bandwidth calculated will be 4 x 516096 = 2580480 kbps / 1024 / 1024 to give you the value in Gbit/s.  Final value = 2.4 Gbit/s.  Full function code = AU4C2GBx

| Bandwidth | Value (kbps) |
| --- | --- |
| STM-4 | 516096 |
| VC12 | 2240 |
| VC4 | 150336 |
| VC3 | 43008 |
| VC2 | 6912 |

Table 60 Bandwidth to Value Mapping

## Circuit Status

Circuit Status

Planned

Installed

Tested

In Service

Faulty

Decommissioned

Table 61 Circuit Status

## Circuit Service Type

Circuit Service Type

ABIS

IuB

IuR

IuCS

IuPS

ATER

Gb

Gi

STP

SS7

A

Table 62 Circuit Service Type

## Rack Type Code List

| Rack Vendor | Rack Model | Rack Type Code |
| --- | --- | --- |
| Hi Tech | Hi Tech 47U | HT47U |

Table 63 Rack Type List

## ODF / Patch Panel Type Code List

| ODF Vendor | ODF Model | ODF Type Code |
| --- | --- | --- |
| ADC | ADC Krone | ADCKR |
|  | ADC FL2000 | ADCFL |
|  | ADC FAME 19’’ | ADCFM |
| LISA | Lisa 19’’ | LIS19 |
| 3M |  |  |
| Fame Frame |  |  |

Table 64 ODF / Patch Panel Type List

## Cable Type Code List

| Model | Type Code |
| --- | --- |
| G652D | G652D |
| G652E | G652E |
| G655D | G655E |
| G655E | G655E |
| G656D | G656D |
| G656E | G656E |
| Hybrid G652D(24)/G656E(24) | G65DE |
| Hybrid G652D/G655D | G65DD |

Table 65 Cable Type List

## Cable Manufacturer List

Supplier

Samsung

CBI

OFS

Corning

Table 66 Cable Manufacturer List

## Civil works/Fibre Build Contractor List

| Civil Works | Abbreviations |
| --- | --- |

| Dark Fibre Africa | DFA |
| --- | --- |
| MCT Telecommunications | MCT |
| Fibrenet | FNI |
| Plessey | PLS |
| Huawei | HWI |
| Yonke | YNK |
| Tovani | TVN |
| Hezeki | HZK |
| XUMA | XMA |
| AME | AME |
| Pwertech (was TSI) | TSI |
| GIO | GIO |
| FibreOp | FOP |
| Sukumani | SKM |

Table 67 Cable Supplier List

## Fibre Size Code List

Fibre Sizes

4F

12F

24F

48F

72F

96F

Table 68 Fibre Size List

## Network Type Code List

| Network Type | Network Type Code |
| --- | --- |
| National Long haul | N |
| Regional Long haul | R |
| Core Fibre | C |
| Access Fibre | A |
| Access Spur | S |
| Fibre Entry | E |

Table 69 Network Type List

## Cable Owner List

Cable Owner

Dark Fibre Africa

Vodacom

City of Johannesburg

City of Tshwane

City of Cape Town

City of Durban

City of Port Elizabeth

Transtel

MTN

Neotel

BBI (Broad Band Infraco)

Others Add cable owner option

Table 70 Cable Owner List

## Cable Request Type

Cable Request Types

LOS Lease

Cable Purchase

Co-Build

Self-Build

Flat Rate Lease/Site Lease

Duct Swap

Fibre Swap

Entry Lease

Table 71 Cable Request Type

## Duct Type

| Duct Type | Duct Code |
| --- | --- |
| 1 Way (32/26mm) Blue Stripe | 1W32BS |
| 1 Way (32/26mm) Red Stripe | 1W32RS |
| 1 Way (50mm) | 1W50mm |
| 2 Way (26mm) | 2W26mm |
| 4 Way (47mm) | 4W47mm |
| 7 Way (50 mm) | 7W50mm |

Table 72 Duct Type List

## Rack Owner

Rack Owner

Vodacom

Telkom

MTN

Cell C

Neotel

Third Party

Teraco

Fibreco

Internet Solution

DFA

Table 73 Rack Owner

## Manhole Type Attribute List

| Manhole Type Attribute | Manhole Name |
| --- | --- |
| Draw box (600 x 600 x1000) | Draw box 600x600x1000 |
| Hand hole frame and lid (Include lock fitment) | Hand hole frame and lid Include lock fitment |
| Vanstone hand hole (standard) – 600 x 450 Manhole: Inside (600 x 450x 500 mm deep); External (680 x 530 x 650 mm deep); Wall thickness (40 mm); Holes (7 of 110mm dia in long side and 4 of 110mm dia in short side) | Vanstone hand hole Standard 600x450x500 |
| Vanstone hand hole (Split bottom) – 600 x 450 Manhole: Inside (600 x 450x 500 mm deep); External (680 x 530 x 650 mm deep); Wall thickness (40 mm); Holes (7 of 110mm dia in long side and 4 of 110mm dia in short side) | Vanstone hand hole Split Bottom 600x450x500 |
| Vanstone hand hole (shallow) | Vanstone hand hole shallow |
| Vanstone Surface Draw Box with Removable Lid | Vanstone Surface Draw Box with Removable Lid |
| Concrete Manhole 1000ID with D700 DMC/SMC Frame & Lid (Suppliers code-GLA00025-A50) | Concrete Manhole 1000ID with D700 DMC or SMC Frame and Lid |
| Type is an Infraset 1 meter Round Chamber, depth of 1.5 metres, ground Coverage of 300mm. Installed on a dry concrete base [ thickness of 200 mm] | Infraset 1m Round Chamber depth of 1.5m ground Coverage of 300mm |

Table 74 Manhole Type List

## Pole Manufacturer and Specification

| Pole Manufacturer | Pole Specification |
| --- | --- |
| Rocla | 9m Hollow 5,5kN |
| Rocla | 9m Cast pole 4kN |
| Rocla | 9m Cast pole 6kN |
| Rocla | 10m Cast pole 8kN |
| Rocla | 11m Cast poles 8kN |
| Aveng | 7m Cast pole 4kN |
| Aveng | 7.2m Cast pole 10kN |
| Aveng | 7.5m Cast pole 20kN |
| Aveng | 9m Cast pole 4kN |
| Aveng | 9m Cast pole 7kN |
| Aveng | 9.3m Cast pole 17.5kN |
| Aveng | 10m Cast pole 8kN |
| Aveng | 11m Cast pole 8kN |

Table 75 Pole Manufacturer and Specification List

## Vodacom Counties

| SUBREGION | COUNTY | COUNTY_ID |
| --- | --- | --- |
| CEN | Bloem and Aliwal | CEN_BA |
| CEN | Northern Cape | CEN_NC |
| CEN | Welkom and Harrismith | CEN_WH |
| EAS | Bay | EAS_BA |
| EAS | Eden | EAS_ED |
| EAS | Kei | EAS_KE |
| KZN | Durban | KZN_DU |
| KZN | North | KZN_NO |
| KZN | South | KZN_SO |
| LES | Lesotho | LES_LS |
| LIM | Capricorn | LIM_CA |
| LIM | Vhembe | LIM_VH |
| LIM | Waterberg | LIM_WA |
| MPU | Ehlanzeni | MPU_EH |
| MPU | Gert Sibande | MPU_GS |
| MPU | Nkangala | MPU_NK |
| NGA | Metro | NGA_ME |
| NGA | North | NGA_NO |
| NGA | North West | NGA_NW |
| SGC | Central | SGC_CE |
| SGC | East | SGC_EA |
| SGC | West | SGC_WE |
| SGS | East | SGS_EA |
| SGS | South | SGS_SO |
| SGS | West | SGS_WE |
| WES | East | WES_EC |
| WES | Rural | WES_RC |
| WES | West | WES_WC |

Table 76 Vodacom Counties

## Physical Trunk Types

Physical Trunk Type

Ethernet

Ethernet 100M

Ethernet 10G

Ethernet 1G

1/0 2M

1/0 8M

OTM

4/4 iSTM-4

4/4 STM-1

4/4 STM-16

4/4 STM-4

4/4 STM-64

Table 77 Trunk Types

## Broadband Interface Types

Broadband Interface Type

Optical

Electrical

Not Applicable

Table 78 Broadband Interface Types

## Product Specification for Cable Labels

| Product | Specification | Usage |
| --- | --- | --- |
| Brother Machine | P-Touch Series Machines | This will be used to print labels to mark OSP and Transmission (ODF) entities in the Vodacom Network |
| Brother Tape | 24mm Brother tape Colour: Black printing on Red Tape | Different font sizes will be printed for different entities. |
| Dartaq Label Holders |  | Only used when labelling OSP plant e.g. Bare Fibres on cable trays in riser ducts and Fibres hauled/floated into 32mm /micro ducts in Hand holes/Manholes. |
| Genuine Grafoplast® TRASP® Marking Elements | Colour: Black Letters on White | This will be used to label the optic cables and optic tails which are changed regularly like patch cables (tails) between devices |
| Fastening Method 1: 3M™ Hook/Loop Fastener | Size as per regional requirement | To fasten the lables to the Optic Tails |
| Fastening Method 2: Cable Tie | Size as per regional requirement | To fasten the labels to complete fibre cables |

Example of Brother Tape Label:

Example of Graftoplast Label:

Table 79 Product Specification for Cable Labels

## Power Components to be labelled:

| Power Type | Power Label |
| --- | --- |
| A-Feed | A-Feed1 (Number 1 to 12) |
| B-Feed | B-Feed1 (Number 1 to 12) |
| Earthing Bar | Label to be visible where it enters the floor: |
| Circuit Breaker | The Label must show which circuit or appliance the Circuit Breaker controls. |
| Shelf Types | Label as: |
|  | Photonic Shelf |
|  | OTN Shelf |
|  | OTS Shelf |
|  | Flex Shelf |
| Fan | Fan |
| Filters | Filter |

Table 80 Broadband Interface Types

## Vodacom Manhole and/or Pole Label Specification:

| Product | Specification | Usage |
| --- | --- | --- |
| Aluminium plates | Anodized aluminium marker plates 80 x 60 mm | Plate to be bonded on the Manhole or Pole with a number to mark the Manhole or Pole for Inventory and Asset Management |
| Bonding Tape or Epoxy | 3M™ VHB™ Tape 4941 or Epoxy to bond Aluminium to Cement | The aluminium marker plates are to be attached to the Cement Pole 2 meters from the ground up. |
| Cleaning Hand Pad | 3M Scotch-Brite General Purpose Hand Pad 7447 (to clean surface) | To clean the Cement Pole surface before the Epoxy or Bonding Tape are applied. |
| Tape Primer | 3M Tape Primer 94 | This will be used to label the which are changed regularly like patch cables (tails) between devices |
| Universal Stamping Kit | 3/8” 10MM Letter & Number Punch Stamp Set Metal-Steel-Hand A-Z *BIG SIZE* 36Pcs | The number is punched-in with the ‘universal stamping kit’ specially designed to ensure that the numbering is perfectly aligned. |

Table 82 Vodacom Manhole or Pole Label Specification

Example and dimensions of anodized aluminium marker plates

Note:  The Plate should be 2mm thick

a.  The Manhole or Pole identification number consists out of 3 x ‘pre-printed’ letters unique to the region e.g. LVP and a 5 digit number identifying the Pole number in that specific region.

b. The Manhole or Pole number is punched-in with the ‘universal stamping kit’ specially designed to ensure that the numbering is perfectly aligned.

c. The aluminum marker plates are attached to the Manhole or Pole with 3M VHB bonding tape.